Apple SCSI Scanners

ERS

Version 1.0

Written by: Antoine Vignau

© Copyright Brutal Deluxe Software, 2024

https://www.brutaldeluxe.fr/

Contents

Technical specifications	4

Apple Scanner (A9M0337)	4

Apple OneScanner (M1381)	4

Apple Color Scanner (BO993LL/A)	4

Apple Color OneScanner 600/27 (M4496LL/A)	4

Apple Color OneScanner 1200/30 (M4495LL/A)	5

SCSI commands	6

TEST UNIT READY ($00)	6

REQUEST SENSE ($03)	7

REQUEST SENSE Return Structure Description for the Apple Scanner	8

REQUEST SENSE Return Structure Description for the OneScanner	10

REQUEST SENSE Return Structure Description for the Color OneScanner	12

INQUIRY ($12)	14

Standard INQUIRY data	15

INQUIRY Return Structure Description for the Apple Scanner	18

INQUIRY Return Structure Description for the OneScanner	20

INQUIRY Return Structure Description for the Color OneScanner	22

INQUIRY Return Structure Description for the Color OneScanner 600/27	24

INQUIRY Return Structure Description for the Color OneScanner 1200/30	25

MODE SELECT ($15)	26

Apple-Specific Data Page Description for the Apple Scanner	27

Apple-Specific Data Page Description for the OneScanner	28

Apple-Specific Data Page Description for the Color OneScanner	29

Disconnect-Reconnect Parameter Page Description	30

Measurement Units Parameter Page Description	31

RESERVE UNIT ($16)	32

RELEASE UNIT ($17)	34

MODE SENSE ($1A)	35

Apple-Specific Data Page Description for the Apple Scanner	36

Apple-Specific Data Page Description for the OneScanner	37

Apple-Specific Data Page Description for the Color OneScanner	38

SCAN ($1B)	39

SCAN Command Structure for the Apple Scanner	39

SCAN Command Structure for the OneScanner	40

SCAN Command Structure for the Color OneScanner series	41

Window Parameter List	41

SEND DIAGNOSTIC ($1D)	42

DEFINE WINDOW PARAMETERS ($24)	43

DEFINE WINDOW PARAMETERS Command Structure for the Apple Scanner	43

DEFINE WINDOW PARAMETERS Command Structure for the OneScanner and the Color OneScanner series	44

DEFINE WINDOW PARAMETERS Parameter List for the Apple Scanner	45

DEFINE WINDOW PARAMETERS Parameter List for the OneScanner	47

DEFINE WINDOW PARAMETERS Parameter List for the Color OneScanner	49

DEFINE WINDOW PARAMETERS Parameter List for the Color OneScanner 600/27	51

DEFINE WINDOW PARAMETERS Parameter List for the Color OneScanner 1200/30	54

Using Window Descriptors With the Apple Scanner	57

Structure details	58

GET WINDOW PARAMETERS ($25)	59

READ ($28)	61

READ Command Structure for the Apple Scanner	61

READ Command Structure for the OneScanner	62

READ Command Structure for the Color OneScanner	63

Structure details	65

SEND ($2A)	66

SEND Command Structure for the Apple Scanner	66

SEND Command Structure for the OneScanner	67

SEND Command Structure for the Color OneScanner	68

Halftone Parameter Page Description for the Apple Scanner and the OneScanner	69

Halftone matrix patterns	70

Gamma Data Write Format for the Color OneScanner	71

Color Correction Matrix for the Color OneScanner	72

PSRAM Data Write Format for the Color OneScanner	73

OBJECT POSITION ($31)	74

GET DATA BUFFER STATUS ($34)	75

GET DATA BUFFER STATUS Return Structure Description for the Apple Scanner	76

GET DATA BUFFER STATUS Return Structure Description for the OneScanner and the Color OneScanner	77

UNKNOWN COMMAND $D3 ($D3)	78

UNKNOWN COMMAND $D4 ($D4)	79

UNKNOWN COMMAND $D5 ($D5)	80

UNKNOWN COMMAND $D6 ($D6)	81

The programming part	82

Find a scanner device	82

Identify the scanner brand and model	82

Preparing the scanner	83

Scanning a page	83

References	85

Halftoning Theory for ASCII Art	85

Programmer's Guide to Apple Scanners Second Edition	85

SCSI-2 Protocol Reference	85

The Complete Scanner Toolkit Macintosh Edition	85

Glossary	86

# Technical specifications

## Apple Scanner (A9M0337)

| Scanner type | Flatbed |
| --- | --- |
| Maximum document size | 8.5-inch by 14.0-inch |
| Interface | SCSI |
| Composition types | Line Art, Halftone, Grayscale |
| Grayscale | 16 levels (4 bits) |
| Output resolution | 75 - 300 dpi |
| Contrast levels | 8 |
| Brightness levels | 16 |
| Threshold values | 16 |
| Graymap settings | 3 |
| Halftone patterns | Spiral, Bayer; downloaded patterns may be up to 4-by-4 matrix |

## Apple OneScanner (M1381)

| Scanner type | Flatbed |
| --- | --- |
| Maximum document size | 8.5-inch by 14.0-inch |
| Interface | SCSI |
| Composition types | Line Art, Halftone, Grayscale |
| Grayscale | 256 levels (8 bits) |
| Output resolution | 72 - 300 dpi in 1 dpi increments |
| Contrast levels | 256 |
| Brightness levels | 256 |
| Threshold values | 256 |
| Graymap settings | 256 |
| Halftone patterns | 4-by-4 and 8-by-8 Spiral and Bayer patterns built in; downloaded patterns may range from 1-by-2 to 16-by-16 |

## Apple Color Scanner (BO993LL/A)

| Scanner type | Flatbed |
| --- | --- |
| Maximum document size | 8.5-inch by 14.0-inch |
| Interface | SCSI |
| Composition types | Line Art, Grayscale, Bi-level Color mode, Full Color mode |
| Color | 16 million colors (24 bits) |
| Output resolution | 75 - 1200 dpi |
| Contrast levels | 256 |
| Brightness levels | 256 |
| Threshold values | 256 |
| Graymap settings | 256 |
| Halftone patterns | Not supported |

## Apple Color OneScanner 600/27 (M4496LL/A)

| Scanner type | Flatbed |
| --- | --- |
| Maximum document size | 8.5-inch by 11.5-inch |
| Interface | SCSI |
| Composition types | Line Art, Grayscale, Bi-level Color mode, Full Color mode |
| Color | 16 million colors externally (134 million colors externally, 27 bits) |
| Output resolution | 300 by 600 dots per inch |
| Contrast levels | 256 |
| Brightness levels | 256 |
| Threshold values | 256 |
| Graymap settings | 256 |
| Halftone patterns | Not supported |

## Apple Color OneScanner 1200/30 (M4495LL/A)

| Scanner type | Flatbed |
| --- | --- |
| Maximum document size | 8.5-inch by 11.5-inch |
| Interface | SCSI |
| Composition types | Line Art, Grayscale, Bi-level Color mode, Full Color mode |
| Color | 1 billion colors (30 bits) |
| Output resolution | 600 by 1200 dots per inch |
| Contrast levels | 256 |
| Brightness levels | 256 |
| Threshold values | 256 |
| Graymap settings | 256 |
| Halftone patterns | Not supported |

Enable Vital Product data

Some of the scanners contain a vital product page. To get the data, send an INQUIRY command with the EVPD bit set to 1, and the page code equals to $F0. The resulting DStatus data is 12 01 F0 00 F0 00. The length of returned data depends on the scanner:

- Scanner

Not applicable

- OneScanner

Not applicable

- Color OneScanner

Not applicable

- Color OneScanner 600/27, length is $45 and data is

06 F0 02 00 45 01 2C 01 2C 11 02 58 02 58 00 0C

00 0C FF FC 00 00 09 F8 00 00 0D A8 0E 00 00 00

09 F8 00 00 10 68 40 01 01 0D 0A FF DF 60 00 00

03 00 08 C1 88 10 00 00 01 00 08 00 0E 04 01 00

00 00 00 8D 00 00 80 00 00 00 FF 00 00 00 00 00

- Color OneScanner 1200/30, length is $45 and data is

06 F0 02 00 45 01 2C 01 2C 11 04 B0 04 B0 00 0C

00 0C FF FF 00 00 13 F0 00 00 1B 50 0E 00 00 00

13 F0 00 00 20 D0 40 01 01 0D 0A FF DF 60 00 00

03 00 08 C1 88 10 00 00 01 00 08 00 0E 04 01 00

00 00 00 8D 00 02 00 00 00 00 FF 00 00 00 00 00

Notes

The scanners use a default measurement unit divisor of 1200, ie. the 1200ths of an inch.

All values are given in decimal.

# SCSI commands

## TEST UNIT READY ($00)

The TEST UNIT READY command provides a means for the host computer to check whether the scanner is ready for normal operation. If the scanner is in a ready condition, the command reports a GOOD status. If the scanner is not ready, the command sets the SCSI sense data to the cause of the error, and the command is terminated with a CHECK CONDITION status. To retrieve the sense data, issue the REQUEST SENSE command (described later in this chapter). Figure 7-1 shows the format of the TEST UNIT READY command structure.

This command is not a request for self-testing. For more information about internal scanner tests, see “SEND DIAGNOSTIC ($1D),” later in this chapter.

The TEST UNIT READY command provides a means to check if the logical unit is ready. This is not a request for a self-test. If the logical unit would accept an appropriate medium-access command without returning CHECK CONDITION status, this command shall return a GOOD status. If the logical unit cannot become operational or is in a state such that an initiator action (e.g. START UNIT command) is required to make the unit ready, the target shall return CHECK CONDITION status with a sense key of NOT READY.

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($00) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) Reserved |  |  |  |  |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | Allocation length |  |  |  |  |  |  |  |
| 5 | Control |  |  |  |  |  |  |  |

Table 74 defines the preferred responses to the TEST UNIT READY command. Higher-priority responses (e.g. BUSY or RESERVATION CONFLICT) are also permitted.

Table 74 - Preferred TEST UNIT READY responses

| Status | Sense key | ASC and ASCQ |
| --- | --- | --- |
| GOOD | NO SENSE | NO ADDITIONAL SENSE INFORMATION or other valid additional sense code. |
| CHECK CONDITION | ILLEGAL REQUEST | LOGICAL UNIT NOT SUPPORTED |
| CHECK CONDITION | NOT READY | LOGICAL UNIT DOES NOT RESPOND TO SELECTION |
| CHECK CONDITION | NOT READY | MEDIUM NOT PRESENT |
| CHECK CONDITION | NOT READY | LOGICAL UNIT NOT READY, CAUSE NOT REPORTABLE |
| CHECK CONDITION | NOT READY | LOGICAL UNIT IS IN PROCESS OF BECOMING READY |
| CHECK CONDITION | NOT READY | LOGICAL UNIT NOT READY, INITIALIZATION COMMAND REQUIRED |
| CHECK CONDITION | NOT READY | LOGICAL UNIT NOT READY, MANUAL INTERVENTION REQUIRED |
| CHECK CONDITION | NOT READY | LOGICAL UNIT NOT READY, FORMAT IN PROGRESS |

## REQUEST SENSE ($03)

The REQUEST SENSE command instructs the scanner to return error and status information. The SCSI sense data is valid after any operation that returns a CHECK CONDITION status. If another operation returns a CHECK CONDITION status before your program retrieves the current sense data, the scanner overwrites the old sense data with the new data.

Figure 7-2 shows the format of the REQUEST SENSE command. The returned information is sent to the host computer in a return structure appropriate to the type of attached scanner. The return structures for the Apple Scanner, OneScanner, and Color OneScanner are described later in this section.

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($03) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) Reserved |  |  |  |  |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | Allocation length |  |  |  |  |  |  |  |
| 5 | Control |  |  |  |  |  |  |  |

### REQUEST SENSE Return Structure Description for the Apple Scanner

The REQUEST SENSE command returns a structure containing the requested information. Figure 7-3 shows the format of the return structure for the Apple Scanner.

Figure 7-3 The REQUEST SENSE return structure for the Apple Scanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($00) Reserved | (7) |  |  | ($00) Reserved |  |  |  |
| 1 | ($00) Reserved |  |  |  |  |  |  |  |
| 2 | ($00) Reserved |  |  |  | Sense key |  |  |  |
| 3 - 6 | ($00) Reserved |  |  |  |  |  |  |  |
| 7 | Additional sense length |  |  |  |  |  |  |  |
| 8 - 17 | ($00) Reserved |  |  |  |  |  |  |  |
| 18 | Dim light | No light | No home | No limit | Shade error | ROM error | RAM error | CPU error |
| 19 | DIPP error | DMA error | GA1 error | ($00) Reserved |  |  |  |  |
| 20 | Optical gain |  |  |  |  |  |  |  |
| 21 | ($00) Reserved |  |  |  |  |  |  |  |

Here is a list of the fields and bits defined in this return structure:

Reserved	These fields are reserved for future expansion. Set them to 0.

Sense key	Bits 0–3 of byte 2 contain the Sense key. The possible Sense key values are as

follows:

$0 	No sense information: there is no specific sense data to be returned to the 	host computer. This situation occurs after a command executes successfully.

$2 	Not ready: the scanner cannot scan without some user intervention.

$4 	Hardware error: the scanner detects a nonrecoverable hardware fault while 	executing a command or during a self-test; these errors are indicated by the 	setting of the hardware error flags in bytes 18 and 19 of the return data 	block.

$5 	Illegal request: the scanner detects an illegal parameter in the command 	block; the scanner cannot perform a scan and cannot alter any parameter data 	within the scanner.

$6 	Unit attention: the scanner has just been switched on or reset and cannot 	execute any command except REQUEST SENSE or INQUIRY.

$9 	Vendor unique: the scanner detects a vendor unique error, which is identified 	by the setting of the Vendor unique error flags in byte 18 of the return data 	block.

Additional sense length

This field indicates the number of sense data bytes that follow. The value of this field does not include bytes 0–7 of the return structure. It reflects the maximum size possible for a returned structure. The scanner does not adjust the value of this field to accommodate the Transfer length parameter specified in your REQUEST SENSE command.

Hardware error flags

Bytes 18 and 19 in the return structure contain bit flags that indicate the specific nature of a hardware error. Flags that are set to 1 indicate a specific hardware error condition. If the returned Sense key field is set to $4, then one or more of these bits are set to 1.

No light	No light is detected from the fluorescent lamp.

No limit	The scanner was unable to locate the limit switch position. Scanning 			cannot proceed because physical damage to the device may occur.

ROM error	A fatal hardware error has been detected in the ROM (an erroneous 			checksum).

RAM error	A fatal hardware error has been detected in the RAM.

CPU error	A fatal hardware error has been detected in the CPU.

DIPP error	A fatal hardware error has been detected in the document image 				preprocessor (DIPP) IC.

DMA error	A fatal hardware error has been detected in the direct memory access 			(DMA) IC.

GA1 error	A fatal hardware error has been detected in the Gate Array 1 (GA1) IC.

Vendor unique error flags

Byte 18 in the return structure contains bit flags that indicate the specific nature of a vendor unique error. Flags that are set to 1 indicate a specific error condition. If the returned Sense key field is set to $9, then one or more of these bits is set to 1.

Dim light	The fluorescent lamp is functioning, but its output has dropped below 			70 percent and is too low for a scan to be accurate.

No home	The scanner was unable to locate the home position mark. Scanning may 			proceed, but the document may not be scanned properly.

Shade error	Three unsuccessful attempts were made to calibrate the charge-coupled 			device (CCD) array correctly. Scanning may continue, but the results may 		be poor. The cause may be a dim lamp or a component that needs servicing.

Optical gain

Indicates the relative intensity of the lamp by returning the gain by which the optical system must boost the analog amplifier to give correct shading results. (This condition is not an error and does not set the Sense key).

### REQUEST SENSE Return Structure Description for the OneScanner

The REQUEST SENSE command returns a structure containing the requested information. Figure 7-4 shows the format of the return structure for the OneScanner.

Figure 7-4 The REQUEST SENSE return structure for the OneScanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($00) Reserved | (7) |  |  | ($00) Reserved |  |  |  |
| 1 | ($00) Reserved |  |  |  |  |  |  |  |
| 2 | ($00) Reserved |  |  |  | Sense key |  |  |  |
| 3 - 6 | ($00) Reserved |  |  |  |  |  |  |  |
| 7 | ($0E) Additional sense length |  |  |  |  |  |  |  |
| 8 - 17 | ($00) Reserved |  |  |  |  |  |  |  |
| 18 | Dim light | No light | No home | No limit | CCDCG | LRAM | CRAM | ROM |
| 19 | ($00) Reserved |  |  |  | SRAM | CPU | ($00) Reserved |  |
| 20 | Optical gain |  |  |  |  |  |  |  |
| 21 | ($00) Reserved |  |  |  |  |  |  |  |

Here is a list of the fields and bits defined in this return structure:

Reserved	These fields are reserved for future expansion. Set them to 0.

Sense key	Bits 0–3 of byte 2 contain the Sense key. The possible Sense key values are as

follows:

$0 	No sense information: there is no specific sense data to be returned to the 	host computer. This situation occurs after a command executes successfully.

$2 	Not ready: the scanner cannot scan without some user intervention.

$4 	Hardware error: the scanner detects a nonrecoverable hardware fault while 	executing a command or during a self-test; these errors are indicated by the 	setting of the hardware error flags in bytes 18 and 19 of the return data 	block.

$5 	Illegal request: the scanner detects an illegal parameter in the command 	block; the scanner cannot perform a scan and cannot alter any parameter data 	within the scanner.

$6 	Unit attention: the scanner has just been switched on or reset and cannot 	execute any command except REQUEST SENSE or INQUIRY.

$9 	Vendor unique: the scanner detects a vendor unique error, which is identified 	by the setting of the Vendor unique error flags in byte 18 of the return data 	block.

Additional sense length

This field indicates the number of sense data bytes that follow. The value of this field does not include bytes 0–7 of the return structure. It reflects the maximum size possible for a returned structure. The scanner does not adjust the value of this field to accommodate the Transfer length parameter specified in your REQUEST SENSE command.

Hardware error flags

Bytes 18 and 19 in the return structure contain bit flags that indicate the specific nature of a hardware error. Flags that are set to 1 indicate a specific hardware error condition. If the returned Sense key field is set to $4, then one or more of these bits are set to 1.

No light	No light is detected from the fluorescent lamp.

No limit	The scanner was unable to locate the limit switch position. Scanning 			cannot proceed because physical damage to the device may occur.

CCDCG		CCD clock generator is not functioning properly. Unit should be 				repaired.

LRAM		Line RAM has one or more bad bits. Unit should be repaired.

CRAM		Correction RAM has one or more bad bits. Unit should be repaired.

ROM		Bad checksum in the control ROM. Unit should be repaired.

SRAM		General-purpose RAM for the scanner CPU has one or more bad bits. Unit 			should be repaired.

CPU		CPU is not functioning properly. Unit should be repaired.

Vendor unique error flag

Bit 7 of byte 18 in the return structure contains a bit flag that indicates the specific nature of a vendor unique error, in this case, Dim light. If the flag is set, it indicates the specific error condition.

Dim light	The fluorescent lamp is functioning, but its output has dropped below 			70 percent and is too low for a scan to be accurate.

Optical gain

Indicates the relative intensity of the lamp by returning the gain by which the optical system must boost the analog amplifier to give correct shading results. (This condition is not an error and does not set the Sense key).

### REQUEST SENSE Return Structure Description for the Color OneScanner

The REQUEST SENSE command returns a structure containing the requested information. Figure 7-5 shows the format of the return structure for the Color OneScanner.

Figure 7-5 The REQUEST SENSE return structure for the Color OneScanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($00) Reserved | ($70) Current error |  |  | ($00) Reserved |  |  |  |
| 1 | ($00) Reserved |  |  |  |  |  |  |  |
| 2 | ($00) Reserved |  |  |  | Sense key |  |  |  |
| 3 - 6 | ($00) Reserved |  |  |  |  |  |  |  |
| 7 | ($0E) Additional sense length |  |  |  |  |  |  |  |
| 8 - 17 | ($00) Reserved |  |  |  |  |  |  |  |
| 18 | Dim light | No light | No home | No limit | Cal Ckt | PSRAM | SRAM | ROM |
| 19 |  | ($00) Reserved |  |  | ICP | ($00) Reserved |  | Over light | ($00) Reserved |
| 20 | Optical gain |  |  |  |  |  |  |  |
| 21 | ($00) Reserved |  |  |  |  |  |  |  |

Here is a list of the fields and bits defined in this return structure:

Reserved	These fields are reserved for future expansion. Set them to 0.

Current error

This field indicates a current scanner error.

Sense key	Bits 0–3 of byte 2 contain the Sense key. The possible Sense key values are as

follows:

$0 	No sense information: there is no specific sense data to be returned to the 	host computer. This situation occurs after a command executes successfully.

$2 	Not ready: the scanner cannot scan without some user intervention.

$4 	Hardware error: the scanner detects a nonrecoverable hardware fault while 	executing a command or during a self-test; these errors are indicated by the 	setting of the hardware error flags in bytes 18 and 19 of the return data 	block.

$5 	Illegal request: the scanner detects an illegal parameter in the command 	block; the scanner cannot perform a scan and cannot alter any parameter data 	within the scanner.

$6 	Unit attention: the scanner has just been switched on or reset and cannot 	execute any command except REQUEST SENSE or INQUIRY.

$9 	Vendor unique: the scanner detects a vendor unique error, which is identified 	by the setting of the Vendor unique error flags in byte 18 of the return data 	block.

Additional sense length

This field indicates the number of sense data bytes that follow. The value of this field does not include bytes 0–7 of the return structure. It reflects the maximum size possible for a returned structure. The scanner does not adjust the value of this field to accommodate the Transfer length parameter specified in your REQUEST SENSE command.

Hardware error flags

Bytes 18 and 19 in the return structure contain bit flags that indicate the specific nature of a hardware error. Flags that are set to 1 indicate a specific hardware error condition. If the returned Sense key field is set to $4, then one or more of these bits are set to 1.

No light	No light is detected from the fluorescent lamp.

No limit	The scanner was unable to locate the limit switch position. Scanning 			cannot proceed because physical damage to the device may occur.

Cal Ckt	The calibration circuit cannot support normal shading correction. 			Scanning should not proceed.

PSRAM		The correction RAM has one or more bad bits. Scanning should not 				proceed.

SRAM		General-purpose RAM for the scanner CPU has one or more bad bits. Unit 			should be repaired.

ROM		Bad checksum in the control ROM. Unit should be repaired.

ICP		The ICP (image correction processor) is not functioning properly after 			a read/write test of the ICP registers. Sense key $4 should be set 			when the ICP error bit is set to 1. Scanning should not proceed.CPU is 			not functioning properly. Unit should be repaired.

Vendor unique error flag

Byte 18 in the return structure contains bit flags that indicate the specific nature of a vendor unique error. Flags that are set to 1 indicate a specific error condition. If the returned Sense key field is set to $9, then one or more of these bits is set to 1.

Dim light	The fluorescent lamp is functioning, but its output has dropped below 			70 percent and is too low for a scan to be accurate.

Lamp needs replacing.

The white target underneath the scanning bed is dirty.

(Color OneScanner only)

The three mirrors are dirty. (Color OneScanner only)

The aperture plate is dirty. (Color OneScanner only)

The CCD is dirty or bad. (Color OneScanner only)

Scanning may continue, but results may not be satisfactory.

No home	The scanner was unable to locate the home position mark.

Scanning may proceed, but the document may not be scanned properly.

Over light	The fluorescent lamp is too bright for normal scanning.

Scanning may proceed, but the document may not be scanned properly.

Optical gain

Indicates the relative intensity of the lamp by returning the gain by which the optical system must boost the analog amplifier to give correct shading results. (This condition is not an error and does not set the Sense key).

## INQUIRY ($12)

The INQUIRY command returns information about the scanner’s device type, firmware version, conformance to various industry standards, and support for the SCSI command set. The following figure shows the format of the INQUIRY command structure. The scanner returns the requested information in a return structure appropriate to the scanner type. These return structures are described later in this section.

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($12) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) Reserved |  |  |  | EVPD |
| 2 | Page code |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | Allocation length |  |  |  |  |  |  |  |
| 5 | Control |  |  |  |  |  |  |  |

An enable vital product data (EVPD) bit of one specifies that the target shall return the optional vital product data specified by the page code field. If the target does not support vital product data and this bit is set to one, the target shall return CHECK CONDITION status with the sense key set to ILLEGAL REQUEST and an additional sense code of INVALID FIELD IN CDB.

An EVPD bit of zero specifies that the target shall return the standard INQUIRY data. If the page code field is not zero, the target shall return CHECK CONDITION status with the sense key set to ILLEGAL REQUEST and an additional sense code of INVALID FIELD IN CDB.

The page code field specifies which page of vital product data information the target shall return (see 8.3.4).

The INQUIRY command shall return CHECK CONDITION status only when the target cannot return the requested INQUIRY data.

NOTE 64 The INQUIRY data should be returned even though the peripheral device may not be ready for other commands.

If an INQUIRY command is received from an initiator with a pending unit attention condition (i.e. before the target reports CHECK CONDITION status), the target shall perform the INQUIRY command and shall not clear the unit attention condition (see 7.9).

NOTES

65 The INQUIRY command is typically used by the initiator after a reset or power-up condition to determine the device types for system configuration. To minimize delays after a reset or power-up condition, the standard INQUIRY data should be available without incurring any media access delays. If the target does store some of the INQUIRY data on the device, it may return zeros or ASCII spaces (20h) in those fields until the data is available from the device.

66 The INQUIRY data may change as the target executes its initialization sequence or in response to a CHANGE DEFINITION command. For example, the target may contain a minimum command set in its non-volatile memory and may load its final firmware from the device when it becomes ready. After it has loaded the firmware, it may support more options and therefore return different supported options information in the INQUIRY data.

### Standard INQUIRY data

The standard INQUIRY data (see table 45) contains 36 required bytes, followed by a variable number of vendor-specific parameters. Bytes 56 through 95, if returned, are reserved for future standardization.

#### Table 45 - Standard INQUIRY data format

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | Peripheral qualifier |  |  | Peripheral device type |  |  |  |  |
| 1 | RMB | Device-type modifier |  |  |  |  |  |  |
| 2 | ISO version |  | ECMA version |  |  | ANSI-approved version |  |  |
| 3 | AENC | TrmIOP | Reserved |  | Response data format |  |  |  |
| 4 | Additional length |  |  |  |  |  |  |  |
| 5 - 6 | Reserved |  |  |  |  |  |  |  |
| 7 | RelAdr | WBus32 | WBus16 | Sync | Linked | Reserved | CmdQue | SftRe |
| 8 - 17 | Vendor identification |  |  |  |  |  |  |  |
| 16 - 31 | Product identification |  |  |  |  |  |  |  |
| 32 - 35 | Product revision level |  |  |  |  |  |  |  |

The peripheral qualifier and peripheral device type fields identify the device currently connected to the logical unit. If the target is not capable of supporting a device on this logical unit, this field shall be set to 7Fh (peripheral qualifier set to 011b and peripheral device type set to 1Fh). The peripheral qualifier is defined in table 46 and the peripheral device type is defined in table 47.

#### Table 46 - Peripheral qualifier

| Code | Description |
| --- | --- |
| 000b | The specified peripheral device type is currently connected to this logical unit. If the target cannot determine whether or not a physical device is currently connected, it shall also use this peripheral qualifier when returning the INQUIRY data. This peripheral qualifier does not mean that the device is ready for access by the initiator. |
| 001b | The target is capable of supporting the specified peripheral device type on this logical unit; however, the physical device is not currently connected to this logical unit. |
| 010b | Reserved |
| 011b | The target is not capable of supporting a physical device on this logical unit. For this peripheral qualifier the peripheral device type shall be set to 1Fh to provide compatibility with previous versions of SCSI. All other peripheral device type values are reserved for this peripheral qualifier. |
| 1XXb | Vendor-specific |

#### Table 47 - Peripheral device type

| Code | Description |
| --- | --- |
| 00h | Direct-access device (e.g. magnetic disk) |
| 01h | Sequential-access device (e.g. magnetic tape) |
| 02h | Printer device |
| 03h | Processor device |
| 04h | Write-once device (e.g. some optical disks) |
| 05h | CD-ROM device |
| 06h | Scanner device |
| 07h | Optical memory device (e.g. some optical disks) |
| 08h | Medium changer device (e.g. jukeboxes) |
| 09h | Communications device |
| 0Ah - 0Bh | Defined by ASC IT8 (Graphic arts pre-press devices) |
| 0Ch - 1Eh | Reserved |
| 1Fh | Unknown of no device type |

A removable medium (RMB) bit of zero indicates that the medium is not removable. A RMB bit of one indicates that the medium is removable.

The device-type modifier field was defined in SCSI-1 to permit vendor- specific qualification codes of the device type. This field is retained for compatibility with SCSI-1. Targets that do not support this field should return a value of zero.

The usage of non-zero code values in the ISO version and ECMA version fields are defined by the International Organization for Standardization and the European Computer Manufacturers Association, respectively. A zero code value in these fields shall indicate that the target does not claim compliance to the ISO version of SCSI (ISO 9316) or the ECMA version of SCSI (ECMA-111). It is possible to claim compliance to more than one of these SCSI standards.

The ANSI-approved version field indicates the implemented version of this International Standard and is defined in table 48.

#### Table 48 - ANSI-approved version

| Code | Description |
| --- | --- |
| 0h | The device might or might not comply to an ANSI-approved standard. |
| 1h | The device complies to ANSI X3.131-1986 (SCSI-1). |
| 2h | The device complies to this version of SCSI. This code is reserved to designate this standard upon approval by ANSI. |
| 3h - 7h | Reserved |

The asynchronous event notification capability (AENC) bit indicates that the device supports the asynchronous event notification capability as defined in 7.5.5.

a) Processor device-type definition: An AENC bit of one indicates that the processor device is capable of accepting asynchronous event notifications. An AENC bit of zero indicates that the processor device does not support asynchronous event notifications.

b) All other device-types: This bit is reserved.

A terminate I/O process (TrmIOP) bit of one indicates that the device supports the TERMINATE I/O PROCESS message as defined in 6.6.22. A value of zero indicates that the device does not support the TERMINATE I/O PROCESS message.

A response data format value of zero indicates the INQUIRY data format is as specified in SCSI-1. A response data format value of one indicates compatibility with some products that were designed prior to the development of this standard (i.e. CCS). A response data format value of two indicates that the data shall be in the format specified in this International Standard. Response data format values greater than two are reserved.

The additional length field shall specify the length in bytes of the parameters. If the allocation length of the command descriptor block is too small to transfer all of the parameters, the additional length shall not be adjusted to reflect the truncation.

A relative addressing (RelAdr) bit of one indicates that the device supports the relative addressing mode for this logical unit. If this bit is set to one, the linked command (Linked) bit shall also be set to one; since relative addressing can only be used with linked commands. A RelAdr bit of zero indicates the device does not support relative addressing for this logical unit.

A wide bus 32 (Wbus32) bit of one indicates that the device supports 32- bit wide data transfers. A value of zero indicates that the device does not support 32-bit wide data transfers.

A wide bus 16 (Wbus16) bit of one indicates that the device supports 16- bit wide data transfers. A value of zero indicates that the device does not support 16-bit wide data transfers.

NOTE 67 If the values of both the Wbus16 and Wbus32 bits are zero, the device only supports 8-bit wide data transfers.

A synchronous transfer (Sync) bit of one indicates that the device supports synchronous data transfer. A value of zero indicates the device does not support synchronous data transfer.

A linked command (Linked) bit of one indicates that the device supports linked commands for this logical unit. A value of zero indicates the device does not support linked commands for this logical unit.

A command queuing (CmdQue) bit of one indicates that the device supports tagged command queuing for this logical unit. A value of zero indicates the device does not support tagged command queuing for this logical unit.

A soft reset (SftRe) bit of zero indicates that the device responds to the RESET condition with the hard RESET alternative (see 6.2.2.1). A SftRe bit of one indicates that the device responds to the RESET condition with the soft RESET alternative (see 6.2.2.2).

ASCII data fields shall contain only graphic codes (i.e. code values 20h through 7Eh). Left-aligned fields shall place any unused bytes at the end of the field (highest offset) and the unused bytes shall be filled with space characters (20h). Right-aligned fields shall place any unused bytes at the start of the field (lowest offset) and the unused bytes shall be filled with space characters (20h).

The vendor identification field contains eight bytes of ASCII data identifying the vendor of the product. The data shall be left aligned within this field.

NOTE 68 It is intended that this field provide a unique vendor identification of the manufacturer of the SCSI device. In the absence of a formal registration procedure, X3T9.2 maintains a list of vendor identification codes in use. Vendors are requested to voluntarily submit their identification codes to X3T9.2 to prevent duplication of codes (see annex E).

The product identification field contains sixteen bytes of ASCII data as defined by the vendor. The data shall be left-aligned within this field.

The product revision level field contains four bytes of ASCII data as defined by the vendor. The data shall be left-aligned within this field.

#### Vital product data

Implementation of vital product data is optional. The information returned consists of configuration data (e.g. vendor identification, product identification, model, serial number), manufacturing data (e.g. plant and date of manufacture), field replaceable unit data and other vendor- or device-specific data.

The initiator requests the vital product data information by setting the EVPD bit to one and specifying the page code of the desired vital product data (see 8.3.4). If the target does not implement the requested page it shall return CHECK CONDITION status. The a sense key shall be set to ILLEGAL REQUEST and the additional sense code shall be set to INVALID FIELD IN CDB.

NOTES

69 The target should have the ability to execute the INQUIRY command even when a device error occurs that prohibits normal command execution. In such a case, CHECK CONDITION status should be returned for commands other than INQUIRY or REQUEST SENSE. The sense data returned may contain the field replaceable unit code. The vital product data should be obtained for the failing device using the INQUIRY command.

70 This International Standard defines a format that allows device- independent initiator software to display the vital product data returned by the INQUIRY command. For example, the initiator may display the data associated for the field replaceable unit returned in the sense data. The contents of the data may be vendor-specific; therefore, it may not be usable without detailed information about the device.

71 This International Standard does not define the location or method of storing the vital product data. The retrieval of the data may require completion of initialization operations within the device that may induce delays before the data is available to the initiator. Time-critical requirements are an implementation consideration and are not addressed in this International Standard.

### INQUIRY Return Structure Description for the Apple Scanner

The INQUIRY return structure for the Apple Scanner contains a 5-byte header followed by 44 bytes of additional inquiry information, as shown in Figure 7-7.

Figure 7-7 The INQUIRY return structure for the Apple Scanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($06) Device type |  |  |  |  |  |  |  |
| 1 | ($00) Reserved |  |  |  |  |  |  |  |
| 2 | ($02) Version |  |  |  |  |  |  |  |
| 3 | ($0) Reserved |  |  |  | ($2) Response data format |  |  |  |
| 4 | ($2C) Additional length |  |  |  |  |  |  |  |
| 5 - 7 | ($00) Reserved |  |  |  |  |  |  |  |
| 8 - 17 | Vendor identification |  |  |  |  |  |  |  |
| 16 - 31 | Product identification |  |  |  |  |  |  |  |
| 32 - 35 | Revision level |  |  |  |  |  |  |  |
| 36 | ($00) Reserved |  |  |  |  |  |  |  |
| 37 | ($00) Reserved |  |  |  |  |  |  |  |
| 38 | ($00) Group 0 commands opcode |  |  |  |  |  |  |  |
| 39 | ($90) Enabled opcodes: $00, $03 |  |  |  |  |  |  |  |
| 40 | ($00) Enabled opcodes: none |  |  |  |  |  |  |  |
| 41 | ($27) Enabled opcodes: $12, $15, $16, $17 |  |  |  |  |  |  |  |
| 42 | ($34) Enabled opcodes: $1A, $1B, $1D |  |  |  |  |  |  |  |
| 43 | ($01) Groupe 1 commands opcode |  |  |  |  |  |  |  |
| 44 | ($08) Enabled opcodes: $24 |  |  |  |  |  |  |  |
| 45 | ($A0) Enabled opcodes: $28, $2A |  |  |  |  |  |  |  |
| 46 | ($08) Enabled opcodes: $34 |  |  |  |  |  |  |  |
| 47 | ($00) Enabled opcodes: none |  |  |  |  |  |  |  |
| 48 | ($FF) End of block |  |  |  |  |  |  |  |

Here is a list of the fields defined in this return structure:

Device type	This field contains the SCSI standard device code of the scanner, which is $06.

Reserved	These fields are reserved for future expansion. Set them to 0.

Version	This field indicates the version number of this scanner, which is $02, as specified in the ANSI SCSI-2 (X3T9.2/86-109) device interface specification.

Response data format

This field indicates that the inquiry data format is as specified in the ANSI SCSI-2 (X3T9.2/86-109) specification. The value of this field is $02 for the Color OneScanner.

Additional length

This field indicates the number of data bytes that follow. The value of this field is $2C and does not include bytes 0–4 of the return structure. The scanner does not adjust the value of this field to accommodate the Transfer length specified in your INQUIRY command.

Vendor identification

This field contains an ASCII string that identifies the scanner’s vendor. For the Color OneScanner, this string is "APPLE", followed by three spaces.

Product identification

This field contains an ASCII string identifying the scanner’s product name and a model number. For the Color OneScanner, this string is "SCANNER A9M0337", followed by one space.

Revision level

This field contains an ASCII string that identifies the scanner’s firmware revision level. For the Apple Scanner, this string is "0.00".

Group 0 commands

A 0 in this field indicates that at least one of the Group 0 scanning command operation codes (as defined in the ANSI SCSI-2 specification) is supported. Bytes 39–42 contain bit flags indicating the Group 0 commands supported by the Apple Scanner. Each command has a corresponding bit in the appropriate bit flag. That bit is set to 1 if the scanner supports the command, and it is set to 0 if the scanner does not support the command.

Enabled opcodes ($00 through $07)

These bit flags indicate the SCSI commands from $00 to $07 that the Apple Scanner supports. The Apple Scanner supports commands $00 (TEST UNIT READY) and $03 (INQUIRY). Therefore, bits 7 and 4 are set to 1. All other bits are set to 0. The value of the field is $90.

Enabled opcodes ($08 through $0F)

None of these operation codes is supported; all bits are set to 0.

Enabled opcodes ($10 through $17)

These bit flags indicate the SCSI commands from $10 to $17 that the Apple Scanner supports. The Apple Scanner supports commands $12 (INQUIRY), $15 (MODE SELECT), $16 (RESERVE), and $17 (RELEASE). Therefore, bits 5, 2, 1, and 0 are set to 1. All other bits are set to 0. The value of the field is $27.

Enabled opcodes ($18 through $1F)

These bit flags indicate the SCSI commands from $18 to $1F that the Apple Scanner supports. The Apple Scanner supports commands $1A (MODE SENSE), $1B (SCAN), and $1D (SEND DIAGNOSTIC). Therefore, bits 5, 4, and 2 are set to 1. All other bits are set to 0. The value of the field is $34.

Group 1 commands

A value of 1 in this field indicates that at least one of the Group 1 scanning commands (as defined in the ANSI SCSI-2 specification) is supported. Bytes 44–47 contain bit flags indicating the Group 1 commands supported by the Apple Scanner. Each command has a corresponding bit in the appropriate bit flag. That bit is set to 1 if the scanner supports the command, and it is set to 0 if the Color scanner does not support the command.

Enabled opcodes ($20 through $27)

These bit flags indicate the SCSI commands from $20 to $27 that the Apple Scanner supports. The Apple Scanner supports command $24 (DEFINE WINDOW PARAMETERS). Therefore, bit 3 is set to 1. All other bits are set to 0. The value of the field is $08.

Enabled opcodes ($28 through $2F)

These bit flags indicate the SCSI commands from $28 to $2F that the Apple Scanner supports. The Apple Scanner supports commands $28 (READ) and $2A (SEND). Therefore, bits 7 and 5 are set to 1. All other bits are set to 0. The value of the field is $A0.

Enabled opcodes ($30 through $37)

These bit flags indicate the SCSI commands from $30 to $37 that the Apple Scanner supports. The Apple Scanner supports command $34 (GET DATA STATUS). Therefore, bit 3 is set to 1. All other bits are set to 0. The value of the field is $08.

Enabled opcodes ($38 through $3F)

None of these operation codes is supported; all bits are set to 0.

End of block	A value of $FF in this field is used to indicate the end of the block.

### INQUIRY Return Structure Description for the OneScanner

The INQUIRY return structure for the OneScanner contains a 5-byte header followed by 44 bytes of additional inquiry information, as shown in Figure 7-8.

Figure 7-8 The INQUIRY return structure for the OneScanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($06) Device type |  |  |  |  |  |  |  |
| 1 | ($00) Reserved |  |  |  |  |  |  |  |
| 2 | ($02) Version |  |  |  |  |  |  |  |
| 3 | ($0) Reserved |  |  |  | ($2) Response data format |  |  |  |
| 4 | ($2C) Additional length |  |  |  |  |  |  |  |
| 5 | ($20) ROM size |  |  |  |  |  |  |  |
| 6 | ($04) CRAM size |  |  |  |  |  |  |  |
| 7 | ($08) SRAM size |  |  |  |  |  |  |  |
| 8 - 17 | Vendor identification |  |  |  |  |  |  |  |
| 16 - 31 | Product identification |  |  |  |  |  |  |  |
| 32 - 35 | Revision level |  |  |  |  |  |  |  |
| 36 | ($00) Buffer space (MSB) |  |  |  |  |  |  |  |
| 37 | ($20) Buffer space (LSB) |  |  |  |  |  |  |  |
| 38 | ($00) Group 0 commands opcode |  |  |  |  |  |  |  |
| 39 | ($90) Enabled opcodes: $00, $03 |  |  |  |  |  |  |  |
| 40 | ($00) Enabled opcodes: none |  |  |  |  |  |  |  |
| 41 | ($27) Enabled opcodes: $12, $15, $16, $17 |  |  |  |  |  |  |  |
| 42 | ($34) Enabled opcodes: $1A, $1B, $1D |  |  |  |  |  |  |  |
| 43 | ($01) Groupe 1 commands opcode |  |  |  |  |  |  |  |
| 44 | ($08) Enabled opcodes: $24 |  |  |  |  |  |  |  |
| 45 | ($A0) Enabled opcodes: $28, $2A |  |  |  |  |  |  |  |
| 46 | ($48) Enabled opcodes: $31, $34 |  |  |  |  |  |  |  |
| 47 | ($00) Enabled opcodes: none |  |  |  |  |  |  |  |
| 48 | ($FF) End of block |  |  |  |  |  |  |  |

Here is a list of the fields defined in this return structure:

Device type	This field contains the SCSI standard device code of the scanner, which is $06.

Reserved	These fields are reserved for future expansion. Set them to 0.

Version	This field indicates the version number of this scanner, which is $02, as specified in the ANSI SCSI-2 (X3T9.2/86-109) device interface specification.

Response data format

This field indicates that the inquiry data format is as specified in the ANSI SCSI-2 (X3T9.2/86-109) specification. The value of this field is $02 for the OneScanner.

Additional length

This field indicates the number of data bytes that follow. The value of this field is $2C and does not include bytes 0–4 of the return structure. The scanner does not adjust the value of this field to accommodate the Transfer length specified in your INQUIRY command.

ROM size	This field indicates the size, in kilobytes, of the scanner ROM. The value of this field is $20.

CRAM size	This field indicates the size, in kilobytes, of the correction RAM. The value of this field is $04.

SRAM size	This field indicates the size, in kilobytes, of the general storage RAM for the scanner CPU. The value of this field is $08.

Vendor identification

This field contains an ASCII string that identifies the scanner’s vendor. For the OneScanner, this string is "APPLE", followed by three spaces.

Product identification

This field contains an ASCII string identifying the scanner’s product name and a model number. For the OneScanner, this string is "SCANNER II", followed by six spaces.

Revision level

This field contains an ASCII string that identifies the scanner’s firmware revision level. For the Apple Scanner, this string is "2.02".

Buffer space	Bytes 36 and 37 indicate the total amount of buffer memory, in kilobytes, in the attached scanner. Buffer memory is used to store scan image data until the host computer reads the data from the scanner. Together, these two bytes contain a 16-bit value: byte 36 contains the most significant byte; byte 37 contains the least significant byte. The value of byte 36 is $00; the value of byte 37 is $20.

Group 0 commands

A 0 in this field indicates that at least one of the Group 0 scanning command operation codes (as defined in the ANSI SCSI-2 specification) is supported. Bytes 39–42 contain bit flags indicating the Group 0 commands supported by the OneScanner. Each command has a corresponding bit in the appropriate bit flag. That bit is set to 1 if the OneScanner supports the command, and it is set to 0 if the OneScanner does not support the command.

Enabled opcodes ($00 through $07)

These bit flags indicate the SCSI commands from $00 to $07 that the OneScanner supports. The OneScanner supports commands $00 (TEST UNIT READY) and $03 (INQUIRY). Therefore, bits 7 and 4 are set to 1. All other bits are set to 0. The value of the field is $90.

Enabled opcodes ($08 through $0F)

None of these operation codes is supported; all bits are set to 0.

Enabled opcodes ($10 through $17)

These bit flags indicate the SCSI commands from $10 to $17 that the OneScanner supports. The OneScanner supports commands $12 (INQUIRY), $15 (MODE SELECT), $16 (RESERVE), and $17 (RELEASE). Therefore, bits 5, 2, 1, and 0 are set to 1. All other bits are set to 0. The value of the field is $27.

Enabled opcodes ($18 through $1F)

These bit flags indicate the SCSI commands from $18 to $1F that the OneScanner supports. The OneScanner supports commands $1A (MODE SENSE), $1B (SCAN), and $1D (SEND DIAGNOSTIC). Therefore, bits 5, 4, and 2 are set to 1. All other bits are set to 0. The value of the field is $34.

Group 1 commands

A value of 1 in this field indicates that at least one of the Group 1 scanning commands (as defined in the ANSI SCSI-2 specification) is supported. Bytes 44–47 contain bit flags indicating the Group 1 commands supported by the OneScanner. Each command has a corresponding bit in the appropriate bit flag. That bit is set to 1 if the OneScanner supports the command, and it is set to 0 if the Onecanner does not support the command.

Enabled opcodes ($20 through $27)

These bit flags indicate the SCSI commands from $20 to $27 that the OneScanner supports. The OneScanner supports command $24 (DEFINE WINDOW PARAMETERS). Therefore, bit 3 is set to 1. All other bits are set to 0. The value of the field is $08.

Enabled opcodes ($28 through $2F)

These bit flags indicate the SCSI commands from $28 to $2F that the OneScanner supports. The OneScanner supports commands $28 (READ) and $2A (SEND). Therefore, bits 7 and 5 are set to 1. All other bits are set to 0. The value of the field is $A0.

Enabled opcodes ($30 through $37)

These bit flags indicate the SCSI commands from $30 to $37 that the OneScanner supports. The OneScanner supports command $31 (OBJECT POSITION) and $34 (GET DATA STATUS). Therefore, bits 6 and 3 are set to 1. All other bits are set to 0. The value of the field is $48.

Enabled opcodes ($38 through $3F)

None of these operation codes is supported; all bits are set to 0.

End of block	A value of $FF in this field is used to indicate the end of the block.

### INQUIRY Return Structure Description for the Color OneScanner

The INQUIRY return structure for the Color OneScanner contains a 5-byte header followed by 48 bytes of additional inquiry information, as shown in Figure 7-9A.

Figure 7-9A The INQUIRY return structure for the Color OneScanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($06) Device type |  |  |  |  |  |  |  |
| 1 | ($00) Reserved |  |  |  |  |  |  |  |
| 2 | ($02) Version |  |  |  |  |  |  |  |
| 3 | ($0) Reserved |  |  |  | ($2) Response data format |  |  |  |
| 4 | ($30) Additional length |  |  |  |  |  |  |  |
| 5 - 7 | ($00) Reserved |  |  |  |  |  |  |  |
| 8 - 17 | Vendor identification |  |  |  |  |  |  |  |
| 16 - 31 | Product identification |  |  |  |  |  |  |  |
| 32 - 35 | Revision level |  |  |  |  |  |  |  |
| 36 | ($00) Buffer space (MSB) |  |  |  |  |  |  |  |
| 37 | ($80) Buffer space (LSB) |  |  |  |  |  |  |  |
| 38 | ($00) Group 0 commands opcode |  |  |  |  |  |  |  |
| 39 | ($90) Enabled opcodes: $00, $03 |  |  |  |  |  |  |  |
| 40 | ($00) Enabled opcodes: none |  |  |  |  |  |  |  |
| 41 | ($27) Enabled opcodes: $12, $15, $16, $17 |  |  |  |  |  |  |  |
| 42 | ($34) Enabled opcodes: $1A, $1B, $1D |  |  |  |  |  |  |  |
| 43 | ($01) Groupe 1 commands opcode |  |  |  |  |  |  |  |
| 44 | ($0C) Enabled opcodes: $24, $25 |  |  |  |  |  |  |  |
| 45 | ($A0) Enabled opcodes: $28, $2A |  |  |  |  |  |  |  |
| 46 | ($48) Enabled opcodes: $31, $34 |  |  |  |  |  |  |  |
| 47 | ($00) Enabled opcodes: none |  |  |  |  |  |  |  |
| 48 | ($FF) End of block |  |  |  |  |  |  |  |
| 49 | ($40) ROM size |  |  |  |  |  |  |  |
| 50 | ($01) PSRAM size (MSB) |  |  |  |  |  |  |  |
| 51 | ($00) PSRAM size (LSB) |  |  |  |  |  |  |  |
| 52 | ($08) SRAM size |  |  |  |  |  |  |  |

Here is a list of the fields defined in this return structure:

Device type	This field contains the SCSI standard device code of the scanner, which is $06.

Reserved	These fields are reserved for future expansion. Set them to 0.

Version	This field indicates the version number of this scanner, which is $02, as specified in the ANSI SCSI-2 (X3T9.2/86-109) device interface specification.

Response data format

This field indicates that the inquiry data format is as specified in the ANSI SCSI-2 (X3T9.2/86-109) specification. The value of this field is $02 for the Color OneScanner.

Additional length

This field indicates the number of data bytes that follow. The value of this field is $30 and does not include bytes 0–4 of the return structure. The scanner does not adjust the value of this field to accommodate the Transfer length specified in your INQUIRY command.

Vendor identification

This field contains an ASCII string that identifies the scanner’s vendor. For the Color OneScanner, this string is "APPLE", followed by three spaces.

Product identification

This field contains an ASCII string identifying the scanner’s product name and a model number. For the Color OneScanner, this string is "SCANNER III", followed by five spaces.

Revision level

This field contains an ASCII string that identifies the scanner’s firmware revision level. For the Color OneScanner, this string is "3.00".

Buffer space	Bytes 36 and 37 indicate the total amount of buffer memory, in kilobytes, in the attached scanner. Buffer memory is used to store scan image data until the host computer reads the data from the scanner. Together, these two bytes contain a 16 bit value: byte 36 contains the most significant byte; byte 37 contains the least significant byte. The value of byte 36 is $00; the value of byte 37 is $80.

Group 0 commands

A 0 in this field indicates that at least one of the Group 0 scanning command operation codes (as defined in the ANSI SCSI-2 specification) is supported. Bytes 39–52 contain bit flags indicating the Group 0 commands supported by the Color OneScanner. Each command has a corresponding bit in the appropriate bit flag. That bit is set to 1 if the Color OneScanner supports the command, and it is set to 0 if the Color OneScanner does not support the command.

Enabled opcodes ($00 through $07)

These bit flags indicate the SCSI commands from $00 to $07 that the Color OneScanner supports. The Color OneScanner supports commands $00 (TEST UNIT READY) and $03 (INQUIRY). Therefore, bits 7 and 4 are set to 1. All other bits are set to 0. The value of the field is $90.

Enabled opcodes ($08 through $0F)

None of these operation codes is supported; all bits are set to 0.

Enabled opcodes ($10 through $17)

These bit flags indicate the SCSI commands from $10 to $17 that the Color OneScanner supports. The Color OneScanner supports commands $12 (INQUIRY), $15 (MODE SELECT), $16 (RESERVE), and $17 (RELEASE). Therefore, bits 5, 2, 1, and 0 are set to 1. All other bits are set to 0. The value of the field is $27.

Enabled opcodes ($18 through $1F)

These bit flags indicate the SCSI commands from $18 to $1F that the Color OneScanner supports. The Color OneScanner supports commands $1A (MODE SENSE), $1B (SCAN), and $1D (SEND DIAGNOSTIC). Therefore, bits 5, 4, and 2 are set to 1. All other bits are set to 0. The value of the field is $34.

Group 1 commands

A value of 1 in this field indicates that at least one of the Group 1 scanning commands (as defined in the ANSI SCSI-2 specification) is supported. Bytes 44–47 contain bit flags indicating the Group 1 commands supported by the Color OneScanner. Each command has a corresponding bit in the appropriate bit flag. That bit is set to 1 if the Color OneScanner supports the command, and it is set to 0 if the Color OneScanner does not support the command.

Enabled opcodes ($20 through $27)

These bit flags indicate the SCSI commands from $20 to $27 that the Color OneScanner supports. The Color OneScanner supports commands $24 (DEFINE WINDOW PARAMETERS) and $25 (GET WINDOW PARAMETERS). Therefore, bits 2 and 3 are set to 1. All other bits are set to 0. The value of the field is $0C.

Enabled opcodes ($28 through $2F)

These bit flags indicate the SCSI commands from $28 to $2F that the Color OneScanner supports. The Color OneScanner supports commands $28 (READ) and $2A (SEND). Therefore, bits 7 and 5 are set to 1. All other bits are set to 0. The value of the field is $A0.

Enabled opcodes ($30 through $37)

These bit flags indicate the SCSI commands from $30 to $37 that the Color OneScanner supports. The Color OneScanner supports commands $31 (OBJECT POSITION) and $34 (GET DATA STATUS). Therefore, bits 6 and 3 are set to 1. All other bits are set to 0. The value of the field is $48.

Enabled opcodes ($38 through $3F)

None of these operation codes is supported; all bits are set to 0.

End of block	A value of $FF in this field is used to indicate the end of the block.

ROM size	This field indicates the size, in kilobytes, of the scanner ROM. The value of this field is $40, equivalent to 64K decimal.

PSRAM size	This field is a two-byte field that indicates the size, in kilobytes, of the correction RAM. It contains $01 and $00, which are equivalent to 256K decimal.

SRAM size	This field indicates the size, in kilobytes, of the general storage RAM for the scanner CPU. The value of this field is $08.

### INQUIRY Return Structure Description for the Color OneScanner 600/27

The INQUIRY return structure for the Color OneScanner contains a 5-byte header followed by 31 bytes of additional inquiry information, as shown in Figure 7-9B.

Figure 7-9B The INQUIRY return structure for the Color OneScanner 600/27

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($06) Device type |  |  |  |  |  |  |  |
| 1 | ($00) Reserved |  |  |  |  |  |  |  |
| 2 | ($02) Version |  |  |  |  |  |  |  |
| 3 | ($0) Reserved |  |  |  | ($2) Response data format |  |  |  |
| 4 | ($1F) Additional length |  |  |  |  |  |  |  |
| 5 - 6 | ($00) Reserved |  |  |  |  |  |  |  |
| 7 | ($10) Reserved |  |  |  |  |  |  |  |
| 8 - 17 | Vendor identification |  |  |  |  |  |  |  |
| 16 - 31 | Product identification |  |  |  |  |  |  |  |
| 32 - 35 | Revision level |  |  |  |  |  |  |  |
| 36 | ($FF) End of block |  |  |  |  |  |  |  |

Here is a list of the fields defined in this return structure:

Device type	This field contains the SCSI standard device code of the scanner, which is $06.

Reserved	These fields are reserved for future expansion. Set them to 0.

Version	This field indicates the version number of this scanner, which is $02, as specified in the ANSI SCSI-2 (X3T9.2/86-109) device interface specification.

Response data format

This field indicates that the inquiry data format is as specified in the ANSI SCSI-2 (X3T9.2/86-109) specification. The value of this field is $02 for the Color OneScanner 600/27.

Additional length

This field indicates the number of data bytes that follow. The value of this field is $1F and does not include bytes 0–4 of the return structure. The scanner does not adjust the value of this field to accommodate the Transfer length specified in your INQUIRY command.

Vendor identification

This field contains an ASCII string that identifies the scanner’s vendor. For the Color OneScanner 600/27, this string is "CANON", followed by three spaces.

Product identification

This field contains an ASCII string identifying the scanner’s product name and a model number. For the Color OneScanner 600/27, this string is "IX-03035B", followed by seven spaces.

Revision level

This field contains an ASCII string that identifies the scanner’s firmware revision level. For the Color OneScanner 600/27, this string is "1.01".

End of block	A value of $FF in this field is used to indicate the end of the block.

### INQUIRY Return Structure Description for the Color OneScanner 1200/30

The INQUIRY return structure for the Color OneScanner contains a 5-byte header followed by 31 bytes of additional inquiry information, as shown in Figure 7-9C.

Figure 7-9C The INQUIRY return structure for the Color OneScanner 1200/30

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($06) Device type |  |  |  |  |  |  |  |
| 1 | ($00) Reserved |  |  |  |  |  |  |  |
| 2 | ($02) Version |  |  |  |  |  |  |  |
| 3 | ($0) Reserved |  |  |  | ($2) Response data format |  |  |  |
| 4 | ($1F) Additional length |  |  |  |  |  |  |  |
| 5 - 6 | ($00) Reserved |  |  |  |  |  |  |  |
| 7 | ($10) Reserved |  |  |  |  |  |  |  |
| 8 - 17 | Vendor identification |  |  |  |  |  |  |  |
| 16 - 31 | Product identification |  |  |  |  |  |  |  |
| 32 - 35 | Revision level |  |  |  |  |  |  |  |
| 36 | ($FF) End of block |  |  |  |  |  |  |  |

Here is a list of the fields defined in this return structure:

Device type	This field contains the SCSI standard device code of the scanner, which is $06.

Reserved	These fields are reserved for future expansion. Set them to 0.

Version	This field indicates the version number of this scanner, which is $02, as specified in the ANSI SCSI-2 (X3T9.2/86-109) device interface specification.

Response data format

This field indicates that the inquiry data format is as specified in the ANSI SCSI-2 (X3T9.2/86-109) specification. The value of this field is $02 for the Color OneScanner 1200/30.

Additional length

This field indicates the number of data bytes that follow. The value of this field is $1F and does not include bytes 0–4 of the return structure. The scanner does not adjust the value of this field to accommodate the Transfer length specified in your INQUIRY command.

Vendor identification

This field contains an ASCII string that identifies the scanner’s vendor. For the Color OneScanner, this string is "CANON", followed by three spaces.

Product identification

This field contains an ASCII string identifying the scanner’s product name and a model number. For the Color OneScanner 1200/30, this string is "IX-06015C", followed by seven spaces.

Revision level

This field contains an ASCII string that identifies the scanner’s firmware revision level. For the Color OneScanner 1200/30, this string is "1.07".

End of block	A value of $FF in this field is used to indicate the end of the block.

## MODE SELECT ($15)

The MODE SELECT command passes a parameter block to the scanner. The scanner uses this data to configure its operating parameters. Figure 7-10 shows the format of the MODE SELECT command structure. The information is passed to the scanner in a MODE SELECT parameter list that is appropriate to the parameter type and attached scanner. These parameter lists are described later in this section.

Figure 7-10 The MODE SELECT command structure

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($15) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | PF | ($00) Reserved |  |  | SP |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | Parameter list length |  |  |  |  |  |  |  |
| 5 | Control |  |  |  |  |  |  |  |

Here is a list of the fields and bits used in this command structure:

Operation code

The operation code for this command is $15.

Reserved 	These fields are reserved for future expansion. Set them to 0.

Page format (PF)

The Page format bit indicates that all parameters after the block descriptors are unique to the vendor, as specified in this reference. This bit must always be set to 1.

Save pages (SP)

A save pages (SP) bit of zero indicates the target shall perform the specified MODE SELECT operation, and shall not save any pages. An SP bit of one indicates that the target shall perform the specified MODE SELECT operation, and shall save to a non-volatile vendor-specific location all the savable pages including any sent during the DATA OUT phase.

Parameter list length

This field contains the length, in bytes, of the parameter list to be passed to the scanner. A transfer length of 0 indicates that no parameter list is being passed (this is not an error condition).

MODE SELECT Parameter List Description

The MODE SELECT parameter list contains one or more optional parameter pages. The parameter page may be either an Apple-specific parameter page or a disconnect-reconnect parameter page. Each parameter page includes 2 bytes that specify the type and the length of the data page. The data pages are described next.

### Apple-Specific Data Page Description for the Apple Scanner

Figure 7-11 shows the Apple-specific data page for the Apple Scanner.

Figure 7-11 The MODE SELECT Apple-specific data page for the Apple Scanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($00) Reserved |  |  |  |  |  |  |  |
| 1 | ($00) Reserved |  |  |  |  |  |  |  |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | ($00) Page code |  |  |  |  |  |  |  |
| 5 | ($06) Page length |  |  |  |  |  |  |  |
| 6 | ($00) Reserved |  |  |  |  |  | Graymap |  |
| 7 | Auto background-adjustment threshold |  |  |  |  |  |  |  |
| 8 | ($00) Reserved |  |  |  |  |  |  | Lamp |
| 9 | ($00) Reserved |  |  |  |  |  |  |  |
| 10 | ($00) Reserved |  |  |  |  |  |  |  |
| 11 | ($00) Reserved |  |  |  |  |  |  |  |

Here is a list of the fields and bits used in this page:

Reserved	These fields are reserved for future expansion. They are set to 0.

Page code	This field identifies the page type. Set them to 0.

Page length	This field contains the length, in bytes, of the data portion of the parameter page, not including bytes 0–5. This field is set to 6.

Graymap 	This field contains the value that controls the graymap function of the scanner. The graymap function allows the scanner to enhance detail in light areas or dark areas of a document, or not to enhance either. A value of 0 results in more detail in the darker areas of the document. A value of 1 results in no alteration of the data. A value of 2 results in more detail in the lighter areas of the document.

Auto background-adjustment threshold
This field indicates the threshold level at which the scanner determines whether a dot is black or white when the user selects automatic back- ground adjustment. The default threshold level is 64.

When excessively dark areas of an original document are scanned, automatic background adjustment constantly adjusts the brightness level, resulting in more detail in the darker areas. Light areas are left unaltered. To select automatic background adjustment, set the Threshold parameter to 0 in the DEFINE WINDOW PARAMETERS command parameter list. See “DEFINE WINDOW PARAMETERS ($24)” later in this chapter.

Lamp 	This bit controls the fluorescent lamp used to illuminate the document during scanning. A value of 1 turns on the lamp. A value of 0 turns it off.

In addition, other conditions may affect whether the lamp is on or off. Once the lamp is turned on, it remains on until two minutes elapse without SCSI activity. The next SCSI command turns on the lamp for a minimum period of two minutes. Furthermore, if the No home bit is set to 1 in the SCAN command, the scanner turns off the lamp only after the carriage assembly returns to the home position. See “SCAN ($1B),” later in this chapter.

### Apple-Specific Data Page Description for the OneScanner

Figure 7-12 shows the Apple-specific data page for the OneScanner.

Figure 7-12 The MODE SELECT Apple-specific data page for the OneScanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($00) Reserved |  |  |  |  |  |  |  |
| 1 - 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | ($00) Page code |  |  |  |  |  |  |  |
| 5 | ($06) Page length |  |  |  |  |  |  |  |
| 6 | ($00) Reserved |  |  |  |  |  |  |  |
| 7 | ($00) Reserved |  |  |  |  | LED | Reset | Button |
| 8 | ($00) Reserved |  |  |  | Fast H | Fast L | CCD | Lamp |
| 9 | ($00) Reserved |  |  |  |  |  |  |  |
| 10 | ($00) Reserved |  |  |  |  |  |  |  |
| 11 | ($00) Reserved |  |  |  |  |  |  |  |

Here is a list of the fields and bits used in this page:

Reserved	These fields are reserved for future expansion. They are set to 0.

Page code	This field identifies the page type. Set it to 0.

Page length	This field contains the length, in bytes, of the data portion of the parameter page, not including bytes 0–5. This field is set to 6.

LED 	This bit indicates the setting of the amber LED on the OneScanner. A value of 1 indicates that the LED is on. A value of 0 indicates that the LED is off.

Reset	This bit indicates whether the scanner should reset the Button bit. A value of 1 causes the scanner to set the Button bit to 0. A value of 0 has no effect.

Button	This bit indicates the current state of the button on the OneScanner since the last reset request (a MODE SELECT command with the Reset bit
set to 1). A value of 1 indicates that the button has been pressed. A value of 0 indicates that the button has not been pressed. Applies only to scanners with ROM version 2.03, or earlier.

Fast H, Fast L

Together, these bits indicate the scan speed for the scanner. Here are four possible values for these bits:

Fast H		Fast L		Scan speed

0 		0 		Normal speed

0		1		High speed

1		0		Fast speed

1		1		Invalid value

At normal speed, the scanner processes the image at the slowest possible carriage speed and performs handshaking on all data exchanges with the host computer. At high speed, the scanner runs the carriage at high speed but still performs handshaking on all data exchanges with the host computer. At normal and high speeds, data handshaking prevents the loss of image data.

At fast speed, the scanner runs the carriage at high speed but does not perform any handshaking during data exchanges with the host. Eliminating the handshaking greatly improves the data exchange rate between the scanner and the host computer. However, image data may be lost if the host computer cannot keep up with the scanner. If the scanner loses data due to an overrun condition, it aborts the scan with a CHECK CONDITION status.

CCD	This bit controls power to the CCD array in the scanner. A value of 1 causes the scanner to power on the CCD array whenever the scanner lamp is turned on. A value of 0 causes the scanner to power on the CCD array only in response to SCAN commands.

Lamp 	This bit indicates the current state of the fluorescent lamp used to illuminate the document during scanning. A value of 1 indicates that the lamp is on; a value of 0 indicates that the lamp is off.

In addition, other conditions may affect whether the lamp is on or off. Once the lamp has been turned on, it remains on until two minutes elapse without SCSI activity. The next SCSI command turns on the lamp for a minimum period of two minutes.

### Apple-Specific Data Page Description for the Color OneScanner

Figure 7-13 shows the Apple-specific data page for the Color OneScanner.

Figure 7-13 The MODE SENSE Apple-specific data page for the Color OneScanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($00) Reserved |  |  |  |  |  |  |  |
| 1 - 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | ($00) Reserved |  | ($00) Apple unique control parameters |  |  |  |  |  |
| 5 | ($06) Parameter length |  |  |  |  |  |  |  |
| 6 | ($00) Reserved |  |  |  |  |  |  |  |
| 7 | ($00) Reserved |  |  |  |  | Scan LED | Gamma reset | 3x3 reset |
| 8 | ($00) Reserved |  |  | MTF circuit | ICP bypass | Data polarity | CCD on | Lamp on |
| 9 | Code sense select |  |  |  |  |  |  |  |
| 10 | ($00) Reserved |  |  |  |  |  |  |  |
| 11 | ($00) Reserved |  |  |  |  |  |  |  |

Here is a list of the fields and bits used in this page:

Reserved	These fields are reserved for future expansion. They are set to 0.

Apple-unique control parameters

Apple unique control parameters are reserved for specific Apple functions. This field is reserved for Page Code, which identifies the page type. Set it to 0.

Page code	This field identifies the page type. Set it to 0.

Parameter length

This field contains the length, in bytes, of the data portion of the parameter page, not including bytes 0–5. This field is set to 6.

Scan LED 	This bit controls the setting of the amber LED on the Color OneScanner. A value of 1 turns on the LED. A value of 0 turns it off.

Gamma reset	When this bit is set (1) the scanner loads the gamma table with default values, which are chosen so that the output data is the same as the input data. The default value is 0.

3x3 reset	This bit directs the scanner to load the default 3-by-3 color correction table. When this bit is set (1), the following default 3-by-3 matrix is loaded. The default value is 0.

1	0	0

0	1	0

0	0	1

MTF circuit	This bit indicates the setting of the MTF circuit.

1	MTF peaking circuit on

0	MTF peaking circuit off

ICP bypass	This bit decides whether the ICP is to be used or bypassed. When the field is set (1) the circuit is bypassed. When it is reset (0) the circuit is used.

Data polarity

This bit determines the polarity of the output image data. When it is
set (1) 00 is white and FF is black. In normal mode, it is reset to 0, and 00 is black and FF white.

CCD on	This bit controls power to the CCD array in the scanner. A value of 1 causes the scanner to power on the CCD array whenever the scanner lamp is turned on. A value of 0 causes the scanner to power on the CCD array only in response to SCAN operations.

Lamp on	This bit controls the fluorescent lamp used to illuminate the document during scanning. A value of 1 turns on the lamp. A value of 0 turns it off.

In addition, other conditions may affect whether the lamp is on or off. Once the lamp has been turned on, it remains on until two minutes elapse without SCSI activity. The next SCSI command turns on the lamp for a minimum period of two minutes.

Color sense select

This field indicates which color sensor line or lines are selected for scanning. The green line sensor is the default for grayscale scanning.

### Disconnect-Reconnect Parameter Page Description

Figure 7-14 shows the disconnect-reconnect parameter page. You may use this page for the Apple Scanner, the OneScanner, and the Color OneScanner.

Here is a list of the fields used in this page:

Reserved	These fields are reserved for future expansion. Set them to 0.

Page code	This field identifies the parameter page as a disconnect-reconnect page.

Set it to $02.

Page length	This field contains the length, in bytes, of the data portion of the parameter page, not including bytes 0–5. Set it to $0A.

Figure 7-14 The MODE SELECT disconnect-reconnect parameter page

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 - 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | ($02) Page code |  |  |  |  |  |  |  |
| 5 | ($0A) Page length |  |  |  |  |  |  |  |
| 6 | Buffer full ratio |  |  |  |  |  |  |  |
| 7 - 15 | ($00) Reserved |  |  |  |  |  |  |  |

Buffer full ratio

Indicates to the scanner how full the scanner’s image buffer must be before the scanner returns image data to the host computer in response to a GET DATA STATUS command (with the Wait bit set to 1). This field provides the numerator of a fraction whose denominator is 255. The resulting ratio controls how full the scanner buffer is allowed to get before data is returned to your program. The recommended value of 128 sets this threshold at 50 percent.

### Measurement Units Parameter Page Description

Figure 7-14B shows the measurement units parameter page. You may use this page for the Color OneScanner 600/27 and the Color OneScanner 1200/30.

Here is a list of the fields used in this page:

Reserved	These fields are reserved for future expansion. Set them to 0.

Page code	This field identifies the parameter page as a measurement units page.

Set it to $03.

Page length	This field contains the length, in bytes, of the data portion of the parameter page, not including bytes 0–5. Set it to $06.

Figure 7-14B The MODE SELECT measurement units page

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 - 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | ($03) Page code |  |  |  |  |  |  |  |
| 5 | ($06) Page length |  |  |  |  |  |  |  |
| 6 | Basic measurement unit |  |  |  |  |  |  |  |
| 7 | ($00) Reserved |  |  |  |  |  |  |  |
| 8 | Measurement unit divisor (MSB) |  |  |  |  |  |  |  |
| 9 | Measurement unit divisor (LSB) |  |  |  |  |  |  |  |
| 10 - 11 | ($00) Reserved |  |  |  |  |  |  |  |

Basic measurement unit

The basic measurement unit field is defined in table below. Targets shall use inches as the default basic measurement unit.

Measurement unit divisor

The measurement unit divisor specifies the number of units needed to equal one basic measurement unit. Targets shall use 1200 as the default measurement unit divisor. If a value of zero is specified the target shall return CHECK CONDITION status and set the sense key to ILLEGAL REQUEST.

Note

A target that does not implement this page or only supports default values uses twelve hundredths (1/1200) of an inch as the unit of measure.

#### Basic measurement units

| Code | Description |
| --- | --- |
| 00h | Inch |
| 01h | Millimètre |
| 02h | Point |
| 03h - FFh | Reserved |

## RESERVE UNIT ($16)

The RESERVE command prevents all devices other than the requesting device from accessing the scanner. The scanner rejects commands that it receives from devices other than the initiating host and instead returns a RESERVATION CONFLICT status. The host computer may reserve the target device for use by a third device by setting the third-party bit (3pty) in byte 1 of the command structure. In this case, only the third device can access the scanner to execute commands, although the initiating host computer may issue another RESERVE command or a RELEASE command. See “RELEASE ($17),” later in this chapter. Figure 7-15 shows the format of the RESERVE command structure.

The RESERVE command is terminated by the RELEASE command or by another RESERVE command issued from the original RESERVE command initiator.

The RESERVE and RELEASE commands together provide the basic mechanism for bus-contention resolution in a multidevice SCSI bus environment. The mechanism works as follows: an application running on a host computer must have uninterrupted access to the scanner while transferring commands and data to and from the scanner. The RESERVE command provides a means for an application to have access to the scanner until it is no longer needed. The application then issues a RELEASE command, which releases the scanner so that another SCSI device on the bus can use it.

Figure 7-15 The RESERVE command structure

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($16) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | 3rdPty | Third party device ID |  |  | ($00) Reserved |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | ($00) Reserved |  |  |  |  |  |  |  |
| 5 | Control |  |  |  |  |  |  |  |

The RESERVE UNIT and RELEASE UNIT commands provide the basic mechanism for contention resolution in multiple-initiator systems.

This command requests that the entire logical unit be reserved for the exclusive use of the initiator until the reservation is superseded by another valid RESERVE UNIT command from the initiator that made the reservation or until released by a RELEASE UNIT command from the same initiator that made the reservation, by a BUS DEVICE RESET message from any initiator, by a hard reset condition, or by a power on cycle. The reservation shall not be granted if the logical unit is reserved by another initiator. It shall be permissible for an initiator to reserve a logical unit that is currently reserved by that initiator.

If the logical unit is reserved for another initiator, the target shall return RESERVATION CONFLICT status.

If, after honouring the reservation, any other initiator attempts to perform any command on the reserved logical unit other than an INQUIRY, REQUEST SENSE, PREVENT ALLOW MEDIUM REMOVAL (with a prevent bit of zero), or a RELEASE UNIT command, the command shall be rejected with RESERVATION CONFLICT status.

Third-party reservation

Third-party reservation allows an initiator to reserve a logical unit for another SCSI device. This is intended for use in multiple- initiator systems that use the COPY command.

If the third-party (3rdPty) bit is zero, a third-party reservation is not requested. If the 3rdPty bit is one the target shall reserve the logical unit for the SCSI device specified in the third-party device ID field. The target shall preserve the reservation until it is superseded by another valid RESERVE UNIT command from the initiator that made the reservation or until it is released by the same initiator, by a BUS DEVICE RESET message from any initiator, or a hard reset condition. The target shall ignore any attempt to release the reservation made by any other initiator.

If independent sets of parameters are implemented, a third party reservation shall cause the target to transfer the set of parameters in effect for the initiator of the RESERVE command to the parameters used for commands from the third-party device. Any subsequent command issued by the third-party device is executed according to the mode parameters in effect for the initiator that sent the RESERVE command.

If independent sets of parameters are implemented, a third party reservation shall cause the target to transfer the set of parameters in effect for the initiator of the RESERVE command to the parameters used for commands from the third party device. Any subsequent command issued by the third-party device is executed according to the mode parameters in effect for the initiator that sent the RESERVE command.

NOTE 147 This transfer of the mode parameters is applicable to target devices which store mode information independently for different initiators. This mechanism allows an initiator to set the mode parameters of a target for the use of a copy master (i.e. the third-party device). The third-party copy master may subsequently issue a MODE SELECT command to modify the mode parameters.

Superseding reservations

An initiator that currently has a logical unit reserved may modify the current reservation by issuing another RESERVE UNIT command to the same logical unit. The superseding reservation shall release the current reservation if the superseding reservation request is granted. The current reservation shall not be modified if the superseding reservation request cannot be granted. If the superseding reservation cannot be granted because of conflicts with a previous reservation (other than the current reservation), then the target shall return RESERVATION CONFLICT status.

NOTE 148 Superseding reservations allow the third-party SCSI device ID to be changed during a reservation using the third-party reservation option. This capability is necessary for certain situations when using COMPARE, COPY, and COPY AND VERIFY commands.

## RELEASE UNIT ($17)

The RELEASE command removes the specified reservation on the scanner. Only the device that issued the original RESERVE command can execute RELEASE. See “RESERVE ($16),” earlier in this chapter, for more information. Figure 7-16 shows the format of the RELEASE command structure.

Figure 7-16 The RELEASE command structure

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($17) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | 3rdPty | Third party device ID |  |  | ($00) Reserved |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | ($00) Reserved |  |  |  |  |  |  |  |
| 5 | Control |  |  |  |  |  |  |  |

The RESERVE UNIT and RELEASE UNIT commands provide the basic mechanism for contention resolution in multiple-initiator systems.

If a valid reservation exists for the I_T_L nexus, the target shall release the reservation and return GOOD status.

A reservation may only be released by the initiator that made it. It is not an error to attempt to release a reservation that is not currently valid or is held by another initiator. In this case, the target shall return GOOD status without altering any other reservation.

Third-party release

Third-party release allows an initiator to release a logical unit that was previously reserved using a third-party reservation (see 10.2.10.1).

If the third party (3rdPty) bit is zero, then a third-party release is not requested. If the 3rdPty bit is one, and if the reservation was made using a third-party reservation by the initiator that is requesting the release for the same SCSI device as specified in the third-party device ID, then the target shall release the reservation.

If the 3rdPty bit is one, the target shall not modify the mode parameters for commands received from the third-party device even if the target implements the transfer of mode parameters with a third-party RESERVE UNIT command.

NOTE 146 When a target implements independent storage of mode parameters for each initiator, a third-party RESERVE UNIT command effects a transfer of the current mode parameters. Those set up by the initiator of the RESERVE UNIT are to be set as the mode parameters used for commands from the third-party device (usually a copy master device). A unit attention condition notifies the third-party device of the changed mode parameters. A successful third-party RELEASE UNIT command leaves the transferred parameters intact. The third-party device can issue MODE SENSE and MODE SELECT commands to query and modify the mode parameters.

## MODE SENSE ($1A)

The MODE SENSE command allows the host computer to read back the parameter information that was sent to the scanner with the MODE SELECT command. See “MODE SELECT ($15),” earlier in this chapter, for more information. Figure 7-17 shows the format of the MODE SENSE command structure. The scanner returns the information to the host computer in one or more MODE SENSE data pages as appropriate for the request and the type of scanner attached. The format of the data pages is described later in this section.

Figure 7-17 The MODE SENSE command structure

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($1A) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | PF | DBD | ($00) Reserved |  |  |
| 2 | PC |  | Page code |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | Parameter list length |  |  |  |  |  |  |  |
| 5 | Control |  |  |  |  |  |  |  |

Here is a list of the fields and bits used in this command structure:

Operation code

The operation code for the MODE SENSE command is $1A.

Reserved	These fields are reserved for future expansion. Set them to 0.

Page format (PF)

This bit indicates that the return data is formatted into pages. Set it to 1.

Disable Block Descriptors (DBD)

A disable block descriptors (DBD) bit of zero indicates that the target may return zero or more block descriptors in the returned MODE SENSE data (see 8.3.3), at the target's discretion. A DBD bit of one specifies that the target shall not return any block descriptors in the returned MODE SENSE data.

Page control (PC)

The page control (PC) field defines the type of mode parameter values to be returned in the mode pages. Here are the three possible values for these bits:

00b	Current values

01b	Changeable values

10b	Default values

11b	Saved values

Page code	This field indicates which type of data page is to be returned.

Set it to 0 to request that the Apple-specific data page appropriate to the attached scanner be returned.

Set it to 2 to request that the disconnect-reconnect data page be returned.

Set it to $3F to request both pages.

Parameter list length

This field contains the expected length, in bytes, of the data to be returned to the host computer. A transfer length of 0 indicates that no return data is to be passed. (This is not an error condition). The scanner stops transferring data when it has returned Parameter list length bytes or all appropriate MODE SENSE data, whichever comes first.

### Apple-Specific Data Page Description for the Apple Scanner

Figure 7-18 shows the Apple-specific data page for the Apple Scanner.

Figure 7-18 The MODE SENSE Apple-specific data page for the Apple Scanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($0B) MODE SENSE data length |  |  |  |  |  |  |  |
| 1 | ($00) Reserved |  |  |  |  |  |  |  |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | ($00) Page code |  |  |  |  |  |  |  |
| 5 | ($06) Page length |  |  |  |  |  |  |  |
| 6 | ($00) Reserved |  |  |  |  |  | Graymap |  |
| 7 | Auto background-adjustment threshold |  |  |  |  |  |  |  |
| 8 | ($00) Reserved |  |  |  |  |  |  | Lamp |
| 9 | ($00) Reserved |  |  |  |  |  |  |  |
| 10 | ($00) Reserved |  |  |  |  |  |  |  |
| 11 | ($00) Reserved |  |  |  |  |  |  |  |

Here is a list of the fields and bits used in this page:

MODE SENSE data length
This field indicates the number of MODE SENSE data bytes that follow for this page. The value is $0B.

Reserved	These fields are reserved for future expansion. They are set to 0.

Page code	This field identifies the page type. It is set to $00.

Page length	This field contains the length, in bytes, of the data portion of the parameter page, not including bytes 0–5. This field is set to 6.

Graymap 	This field contains the value that controls the graymap function of the scanner. The graymap function allows the scanner to enhance detail in light areas or dark areas of a document, or not to enhance either. A value of 0 results in more detail in the darker areas of the document. A value of 1 results in no alteration of the data. A value of 2 results in more detail in the lighter areas of the document.

Auto background-adjustment threshold
This field indicates the threshold level at which the scanner determines whether a dot is black or white when the user selects automatic back- ground adjustment. The default threshold level is 64.

When excessively dark areas of an original document are scanned, automatic background adjustment constantly adjusts the brightness level, resulting in more detail in the darker areas. Light areas are left unaltered. To select automatic background adjustment, set the Threshold parameter to 0 in the DEFINE WINDOW PARAMETERS command parameter list. See “DEFINE WINDOW PARAMETERS ($24)” later in this chapter.

Lamp 	This bit indicates the current state of the fluorescent lamp used to illuminate the document during scanning. A value of 1 indicates that the lamp is on; a value of 0 indicates that the lamp is off.

### Apple-Specific Data Page Description for the OneScanner

Figure 7-19 shows the Apple-specific data page for the OneScanner.

Figure 7-19 The MODE SENSE Apple-specific data page for the OneScanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($0B) MODE SENSE data length |  |  |  |  |  |  |  |
| 1 - 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | ($00) Page code |  |  |  |  |  |  |  |
| 5 | ($06) Page length |  |  |  |  |  |  |  |
| 6 | ($00) Reserved |  |  |  |  |  |  |  |
| 7 | ($00) Reserved |  |  |  |  | Scan LED | Reset | Button |
| 8 | ($00) Reserved |  |  |  | Fast H | Fast L | CCD | Lamp |
| 9 | ($00) Reserved |  |  |  |  |  |  |  |
| 10 | ($00) Reserved |  |  |  |  |  |  |  |
| 11 | ($00) Reserved |  |  |  |  |  |  |  |

Here is a list of the fields and bits used in this page:

MODE SENSE data length
This field indicates the number of MODE SENSE data bytes that follow for this page. The value is $0B.

Reserved	These fields are reserved for future expansion. They are set to 0.

Page code	This field identifies the page type. It is set to $00.

Page length	This field contains the length, in bytes, of the data portion of the parameter page, not including bytes 0–5. This field is set to 6.

Scan LED 	This bit indicates the setting of the amber LED on the OneScanner. A value of 1 indicates that the LED is on. A value of 0 indicates that the LED is off.

Reset	This bit indicates whether the scanner should reset the Button bit. It is used only for the MODE SELECT command. Do not use this bit. Applies only to scanners with ROM version 2.03, or earlier.

Button	This bit indicates the current state of the button on the OneScanner since the last reset request (a MODE SELECT command with the Reset bit set to 1). A value of 1 indicates that the button has been pressed. A value of 0 indicates that the button has not been pressed. Applies only to scanners with ROM version 2.03, or earlier.

Fast H, Fast L

Together, these bits indicate the scan speed for the scanner. Here are the three possible values for these bits:

Fast H		Fast L		Scan speed

0 		0 		Normal speed

0		1		High speed

1		0		Fast speed

1		1		Invalid value

At normal speed, the scanner processes the image at the slowest possible carriage speed and performs handshaking on all data exchanges with the host computer. At high speed, the scanner runs the carriage at high speed but still performs handshaking on all data exchanges with the host computer. At normal and high speeds, data handshaking prevents the loss of image data.

At fast speed, the scanner runs the carriage at high speed but does not perform any handshaking during data exchanges with the host. Eliminating the handshaking greatly improves the data exchange rate between the scanner and the host computer. However, image data may be lost if the host computer cannot keep up with the scanner. If the scanner loses data due to an overrun condition, it aborts the scan with a CHECK CONDITION status.

CCD	This bit controls power to the CCD array in the scanner. A value of 1 causes the scanner to power on the CCD array whenever the scanner lamp is turned on. A value of 0 causes the scanner to power on the CCD array only in response to SCAN commands.

Lamp 	This bit indicates the current state of the fluorescent lamp used to illuminate the document during scanning. A value of 1 indicates that the lamp is on; a value of 0 indicates that the lamp is off.

### Apple-Specific Data Page Description for the Color OneScanner

Figure 7-20 shows the Apple-specific data page for the Color OneScanner.

Figure 7-20 The MODE SENSE Apple-specific data page for the Color OneScanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($0B) MODE SENSE data length |  |  |  |  |  |  |  |
| 1 - 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | ($00) Page code |  |  |  |  |  |  |  |
| 5 | ($06) Page length |  |  |  |  |  |  |  |
| 6 | ($00) Reserved |  |  |  |  |  |  |  |
| 7 | ($00) Reserved |  |  |  |  | Scan LED | ($00) | ($00) |
| 8 | ($00) Reserved |  |  | MTF circuit | ICP bypass | Data polarity | CCD on | Reset lamp on |
| 9 | Code sense select |  |  |  |  |  |  |  |
| 10 | ($00) Reserved |  |  |  |  |  |  |  |
| 11 | ($00) Reserved |  |  |  |  |  |  |  |

Here is a list of the fields and bits used in this page:

MODE SENSE data length
This field indicates the number of MODE SENSE data bytes that follow for this page. The value is $0B.

Reserved	These fields are reserved for future expansion. They are set to 0.

Page code	This field identifies the page type. It is set to $00.

Page length	This field contains the length, in bytes, of the data portion of the parameter page, not including bytes 0–5. This field is set to 6.

Scan LED 	This bit indicates the setting of the amber LED which indicates the status of certain application functions. The default state for the LED is off.

MTF circuit	This bit indicates the setting of the MTF circuit. If it is set to 1, the circuit is turned on. When it is reset to 0, the circuit is turned off.

ICP bypass	This bit indicates the setting of the ICP. When it is set to 1, the circuit is bypassed. When it is reset to 0, the circuit is used.

Data polarity

This bit indicates the polarity of the output image data. When it is set (1), 00 = white, and FF = black. When it is reset (0), 00 = black, and FF = white. The latter is the normal mode.

CCD on	This bit indicates if the scanner has turned on power to the CCD (charge-coupled device) array when the scanning lamp is on. Whenever the scan command is issued, the CCD is powered up, even if the CCD on bit is set to 0.

Lamp on	This bit, when set, indicates the scanner has turned on the lamp. The scanner goes through a shading correction before each scan, regardless of the state of the Lamp on bit. The lamp remains on until the next MODE SELECT signal is received, and the field is cleared. At this time, the lamp resumes normal on/off operation. If the lamp is on and there is no SCSI activity for more two minutes or more, the lamp turns off. When the scanner receives any SCSI command, the lamp turns on for a minimum of two minutes.

Color sense select

This field indicates which color sensor line or lines are selected for scanning. The green line sensor is the default for grayscale scanning.

## SCAN ($1B)

The SCAN command instructs the scanner to begin the scanning operation. Following the SCAN command structure, you may choose to send a Window identifier byte that identifies the scan area to process. See “DEFINE WINDOW PARAMETERS ($24),” later in this chapter, for more information.

For the Color OneScanner, there are two ways to abort a scan. The first way is to send the ABORT message. The second way is to issue the SCAN command, with the Transfer length parameter set to 0. For Apple Scanners and OneScanners, sending any command will abort the scan.

### SCAN Command Structure for the Apple Scanner

Figure 7-21 shows the format of the SCAN command structure for the Apple Scanner.

Figure 7-21 The SCAN command structure for the Apple Scanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($1B) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) Reserved |  |  |  |  |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | Transfer length |  |  |  |  |  |  |  |
| 5 | Wait | No home | ($00) Reserved |  |  |  |  |  |

Here is a list of the fields and bits used in this command structure:

Operation code

The operation code for the SCAN command is $1B.

Reserved	These fields are reserved for future expansion. Set them to 0.

Transfer length

This field indicates whether the application supplies a Window identifier byte after the 6 command bytes. If the value of the transfer length is 0, the application supplies no Window identifier byte and no scan takes place. If the value of the transfer length is 1, the application supplies a Window identifier byte immediately after the command structure.

The Window identifier byte must correspond to a Window identifier field value supplied to the DEFINE WINDOW PARAMETERS command.

Wait	If this bit is set to 1, the user must press the button on the scanner before the scanner will begin scanning.

No home	If this bit is set to 1, the scanner lamp stays on and the carriage assembly remains where it stops at the end of a scan. After two minutes, if the scanner does not receive another SCAN command, the lamp goes off and the carriage assembly returns to the home position.

### SCAN Command Structure for the OneScanner

Figure 7-22 shows the SCAN command structure for the OneScanner.

Figure 7-22 The SCAN command structure for the OneScanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($1B) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) Reserved |  |  |  |  |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | Transfer length |  |  |  |  |  |  |  |
| 5 | ($00) Reserved |  | Non-Cal | ($00) Reserved |  |  |  |  |

Here is a list of the fields and bits used in this command structure:

Operation code

The operation code for the SCAN command is $1B.

Reserved	These fields are reserved for future expansion. Set them to 0.

Transfer length

This field indicates whether the application supplies a Window identifier byte after the 6 command bytes. If the value of the transfer length is 0, the application supplies no Window identifier byte and no scan takes place. If the value of the transfer length is 1, the application supplies a Window identifier byte immediately after the command structure.

The Window identifier byte must correspond to a Window identifier field value supplied to the DEFINE WINDOW PARAMETERS command.

Non-Cal	Controls whether the scanner calibrates for the current lamp intensity before starting the scan. If this bit is set to 1, the scanner does not perform calibration. If this bit is set to 0, the scanner calibrates for the current lamp intensity before scanning. This calibration step takes a few seconds. Your application can reduce scan time by skipping this calibration step. However, skipping lamp calibration compromises scan quality. Consequently, your application does not use calibration except for preview scans.

### SCAN Command Structure for the Color OneScanner series

Figure 7-23 shows the format of the SCAN command structure for the Color OneScanner.

Figure 7-23 The SCAN command structure for the Color OneScanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($1B) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) Reserved |  |  |  |  |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | Transfer length |  |  |  |  |  |  |  |
| 5 | Control |  |  |  |  |  |  |  |

Here is a list of the fields and bits used in this command structure:

Operation code

The operation code for the SCAN command is $1B.

Reserved	These fields are reserved for future expansion. Set them to 0.

Transfer length

This field indicates whether the application supplies a Window identifier byte after the 6 command bytes. If the value of the transfer length is 0, the application supplies no Window identifier byte and no scan takes place. If the value of the transfer length is 1, the application supplies a Window identifier byte immediately after the command structure.

The Window identifier byte must correspond to a Window identifier field value supplied to the DEFINE WINDOW PARAMETERS command.

### Window Parameter List

To cause the scanner to scan a document, you must append a Window identifier byte to the SCAN command structure. The Window identifier byte specifies the scan area and related scanning parameters, and it must correspond to a window identifier defined with the DEFINE WINDOW PARAMETERS command. Figure 7-24 shows the command byte format.

Figure 7-24 The SCAN command Window identifier byte

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | Window identifier |  |  |  |  |  |  |  |

FIELD DESCRIPTION

Window identifier

Specifies the scan area and related parameters for the scan operation. The value of the identifier byte must be the same as the value of the Window identifier byte indicated in the DEFINE WINDOW PARAMETERS command for that window.

## SEND DIAGNOSTIC ($1D)

The SEND DIAGNOSTIC command directs the scanner to run a built-in self-test. Figure 7-25 shows the format of the SEND DIAGNOSTIC command structure.

Figure 7-25 The SEND DIAGNOSTIC command structure

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($1D) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) PF | ($00) Reserved | SelfTest | ($00) DevOfL | ($00) UnitOfL |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | Parameter list (MSB) |  |  |  |  |  |  |  |
| 4 | Parameter list (LSB) |  |  |  |  |  |  |  |
| 5 | Control |  |  |  |  |  |  |  |

FIELD DESCRIPTIONS

Here is a list of the fields and bits used in this command structure:

Operation code
The operation code for the SEND DIAGNOSTIC command is $1D.

Reserved	These fields are reserved for future expansion. Set them to 0.

Page format (PF)

A page format (PF) bit of one specifies that the SEND DIAGNOSTIC parameters conform to the page structure as specified in this International Standard. The implementation of the PF bit is optional. See 87.3.1 for the definition of diagnostic pages. A PF bit of zero indicates that the SEND DIAGNOSTIC parameters are as specified in SCSI-1 (i.e. all parameters are vendor-specific).

SelfTest	Set this bit to 1 to request a self-test.

DevOfL	A DevOfl bit of one grants permission to the target to perform diagnostic operations that may affect all the logical units on a target, e.g. alteration of reservations, log parameters, or sense data. The implementation of the DevOfl bit is optional. A DevOfL bit of zero prohibits diagnostic operations that may be detected by subsequent I/O processes.

UnitOfL	A UnitOfL bit of one grants permission to the target to perform diagnostic operations that may affect the user accessible medium on the logical unit, e.g. write operations to the user accessible medium, or repositioning of the medium on sequential access devices. The implementation of the UnitOfl bit is optional. A UnitOfL bit of zero prohibits any diagnostic operations that may be detected by subsequent I/O processes.

The device off-line (DevOfL) and unit off-line (UnitOfL) bits are generally set by operating system software, while the parameter list is prepared by diagnostic application software. These bits grant permission to perform vendor-specific diagnostic operations on the target that may be visible to attached initiators. Thus, by preventing operations that are not enabled by these bits, the target assists the operating system in protecting its resources.

## DEFINE WINDOW PARAMETERS ($24)

Before the scanner can perform a scan, the application program must provide certain details about the scan area. This information is provided in the form of parameters defining a window for each scan area. The DEFINE WINDOW PARAMETERS command passes this window-definition data to the scanner. The program defines the size, position, scanning resolution, scanning composition, and other parameters of each window. There are different parameter structures for each supported scanner. These structures are described later in this section.

IMPORTANT

The OneScanner and Color OneScanner do not support multiple windows. ▲

### DEFINE WINDOW PARAMETERS Command Structure for the Apple Scanner

Figure 7-26 shows the format of the DEFINE WINDOW PARAMETERS command structure for the Apple Scanner.

Figure 7-26 The DEFINE WINDOW PARAMETERS command structure for the Apple Scanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($24) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) Reserved |  |  |  |  |
| 2 - 5 | ($00) Reserved |  |  |  |  |  |  |  |
| 6 | Transfer length (MSB) |  |  |  |  |  |  |  |
| 7 | Transfer length |  |  |  |  |  |  |  |
| 8 | Transfer length (LSB) |  |  |  |  |  |  |  |
| 9 | Apple | Control |  |  |  |  |  |  |

FIELD DESCRIPTIONS

Here is a list of the fields and bits used in this command structure:

Operation code

The operation code for the DEFINE WINDOW PARAMETERS command is $24.

Reserved 	These fields are reserved for future expansion. Set them to 0.

Transfer length

This field contains the length, in bytes, of the DEFINE WINDOW PARAMETERS parameter list. A transfer length of 0 indicates that the application program does not pass any parameters (this is not an error condition). The transfer length indicates the total length of all window descriptors and window parameter lists. The total transfer length is

8 + (40 ∗ n) bytes

where n is the number of windows. Byte 6 of the structure contains the most-significant byte of the transfer length; byte 8 contains the least-significant byte of the transfer length.

Apple	This bit, if set to 1, allows the Apple Scanner to recognize multiple window descriptors.

### DEFINE WINDOW PARAMETERS Command Structure for the OneScanner and the Color OneScanner series

Figure 7-27 shows the format of the DEFINE WINDOW PARAMETERS command structure for the OneScanner and the Color OneScanner.

Figure 7-27 The DEFINE WINDOW PARAMETERS command structure for the OneScanner and the Color OneScanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($24) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) Reserved |  |  |  |  |
| 2 - 5 | ($00) Reserved |  |  |  |  |  |  |  |
| 6 | Transfer length (MSB) |  |  |  |  |  |  |  |
| 7 | Transfer length |  |  |  |  |  |  |  |
| 8 | Transfer length (LSB) |  |  |  |  |  |  |  |
| 9 | Control |  |  |  |  |  |  |  |

FIELD DESCRIPTIONS

Here is a list of the fields and bits used in this command structure:

Operation code

The operation code for the DEFINE WINDOW PARAMETERS command is $24.

Reserved 	These fields are reserved for future expansion. Set them to 0.

Transfer length

This field contains the length, in bytes, of the DEFINE WINDOW PARAMETERS parameter list. If the value of the transfer length is 0, the application does not pass any parameters (this is not an error condition). The transfer length indicates the total length of all window descriptors and window parameter lists. The total transfer length for each model is described below. Byte 6 of the structure contains the most-significant byte of the transfer length; byte 8 contains the least-significant byte of the transfer length.

#### Total Transfer Length

| Model | Total Transfer Length (in bytes) |
| --- | --- |
| Apple Scanner | 48 |
| OneScanner | 48 |
| Color OneScanner | 50 |
| Color OneScanner 600/27 | 72 |
| Color OneScanner 1200/30 | 72 |

### DEFINE WINDOW PARAMETERS Parameter List for the Apple Scanner

The window parameter list for the DEFINE WINDOW PARAMETERS command consists of one parameter list header and one or more unique window descriptors. When the user wants to scan more than one window, the application program must supply one window descriptor for each window. Figure 7-28 shows the format of the parameter list header and descriptor.

Figure 7-28 shows the DEFINE WINDOW PARAMETERS parameter list for the Apple Scanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | Data transfer length (MSB) |  |  |  |  |  |  |  |
| 1 | Data transfer length (LSB) |  |  |  |  |  |  |  |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | ($00) Reserved |  |  |  |  |  |  |  |
| 5 | ($00) Reserved |  |  |  |  |  |  |  |
| 6 | Window descriptor length (MSB) |  |  |  |  |  |  |  |
| 7 | Window descriptor length (LSB) |  |  |  |  |  |  |  |

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | Window identifier |  |  |  |  |  |  |  |
| 1 | ($00) Reserved |  |  |  |  |  |  |  |
| 2 - 3 | X-Axis resolution |  |  |  |  |  |  |  |
| 4 - 5 | Y-Axis resolution |  |  |  |  |  |  |  |
| 6 - 9 | X-Axis upper left |  |  |  |  |  |  |  |
| 10 - 13 | Y-Axis upper left |  |  |  |  |  |  |  |
| 14 - 17 | Window width |  |  |  |  |  |  |  |
| 18 - 21 | Window length |  |  |  |  |  |  |  |
| 22 | Brightness |  |  |  |  |  |  |  |
| 23 | Threshold |  |  |  |  |  |  |  |
| 24 | Contrast |  |  |  |  |  |  |  |
| 25 | Image composition |  |  |  |  |  |  |  |
| 26 | Bits per pixel |  |  |  |  |  |  |  |
| 27 - 28 | Halftone pattern |  |  |  |  |  |  |  |
| 29 | ($00) Reserved |  |  |  |  | Padding type |  |  |
| 30 - 31 | ($00) Bit ordering |  |  |  |  |  |  |  |
| 32 | Compression type |  |  |  |  |  |  |  |
| 33 | ($00) Compression argument |  |  |  |  |  |  |  |
| 34 | ($00) Reserved |  |  |  |  |  |  |  |
| 35 | ($00) Reserved |  |  |  |  |  |  |  |
| 36 | ($00) Reserved |  |  |  |  |  |  |  |
| 37 | ($00) Reserved |  |  |  |  |  |  |  |
| 38 | ($00) Reserved |  |  |  |  |  |  |  |
| 39 | ($00) Reserved |  |  |  |  |  |  |  |

FIELD DESCRIPTIONS

The parameter list header describes the length, in bytes, of a window descriptor. An application program may transfer a maximum of 99 window descriptors. Here are the parameter list fields used in this structure:

Reserved 	These fields are reserved for future expansion. Set them to 0.

Parameter list length

This field specifies the length in bytes of a single window descriptor. Always set this field to 40 ($28).

The window descriptor contains information about one window. Here are the window descriptors used in this structure:

Window identifier

This field contains a number between 0 and 255, which uniquely identifies the window defined by the descriptor. Use this unique identifier to identify each window during data transfers and status requests.

Reserved	These fields are reserved for future expansion. Set them to 0.

X resolution	This field specifies the resolution, in dots per inch, along the horizontal direction (x-axis). In Line Art mode and Halftone mode, the Apple Scanner supports resolutions of 300, 285, 270, 255, 240, 225, 210, 200, 195, 180, 165, 150, 135, 120, 105, 100, 90, and 75 dpi. In Grayscale mode, the Apple Scanner supports resolutions of 300, 200, 150, 100, and 75 dpi. A value of 0 in this field indicates that the scanner uses a default horizontal resolution value of 75 dpi.

Y resolution This field specifies the resolution, in dots per inch, in the vertical direction (y-axis). In Line Art mode and Halftone mode, the Apple Scanner supports resolutions of 300, 285, 270, 255, 240, 225, 210, 200, 195, 180, 165, 150, 135, 120, 100, 90, and 75 dpi. In Grayscale mode, the Apple Scanner supports resolutions of 300, 200, 150, 100, and 75 dpi. A value of 0 in this field indicates that the scanner uses a default vertical resolution value of 75 dpi.

Upper-left x This field specifies the x-coordinate of the upper left corner of the rectangular window to be scanned. The point (0,0) is considered the upper-left corner of the window. Coordinates are measured from this point in increments of 1/1200 of an inch from the right edge of the scanner’s glass surface (as viewed when you face the glass). Coordinates must be multiples of 8 times the x resolution; they must lie on byte boundaries. The default value of this field is 0.

Upper-left y This field specifies the y-coordinate of the upper-left corner of the rectangular window to be scanned. The point (0,0) is considered the upper-left corner of the window. Coordinates are measured from this point in increments of 1/1200 of an inch from the top edge of the scanner’s glass surface. The default value of this field is 0.

Width	This field specifies the window width in increments of 1/1200 of an inch along the horizontal axis. This value must be a multiple of 8 times the x resolution; the window border must lie on a byte boundary. The default value of this field is 10,208.

Length	This field specifies the window length in increments of 1/1200 of an inch along the vertical axis. The default value of this field is 13,200.

Brightness	This field specifies the brightness. A value of 0 results in the default value of 128. Any other value indicates a relative brightness: 255 is the highest setting, 1 is the lowest setting, and 128 is the average setting.

Threshold	This field specifies the threshold level. A value of 0 in Line Art mode causes the scanner to use automatic background adjustment. See “MODE SELECT ($15),” earlier in this chapter, for a description of the Auto background-adjustment threshold field. A value of 0 received in any other mode results in the use of the default setting. Any other value indicates a relative threshold parameter: 255 is the highest setting, 1 is the lowest setting, and 128 is the average setting. The default value for this parameter is 128.

Contrast	This field specifies the contrast at which the scanner scans the document. A value of 0 indicates that the scanner uses the default value of 1. Any other value indicates a relative contrast: 255 is the highest setting, 1 is the lowest setting, and 128 is the average setting. The contrast setting is valid only in Halftone mode and Grayscale mode.

Image composition

This field specifies the type of image data acquired. The default composition mode is Line Art mode. The following values are valid:

$00  Line Art mode

$01  Halftone mode

$02  Grayscale mode

Bits per pixel

This field specifies, for any one pixel, the number of bits used to specify the visible density of the document being scanned. This field is valid only if the Image composition field indicates that gray-scale data is desired. The Apple Scanner supports 16 levels of gray in Grayscale mode and only black and white in Line Art mode and Halftone mode. Therefore, 4 bits per pixel are required to represent the full range of grays, and only 1 bit per pixel is required to represent either black or white.

Halftone pattern

This field specifies the halftone process by which the scanner converts multilevel gray tones to black and white dots. This parameter is only valid when the Image composition field specifies that halftone image data is desired. The following values are valid:

$00  Spiral

$01  Bayer

$02  Downloaded pattern

Padding type This field specifies what the scanner does if the image data transmitted to the host is not a whole number of bytes. This field must be $03, which truncates the data to the next byte boundary.

Bit ordering	This field is resersed. Set the value to $0000.

Compression type

This field specifies the compression technique that the scanner applies to the image data prior to transmission to the host computer. The default is no compression. The following values are valid:

$00  No compression

$01  CCITT Group III, one-dimensional

$83 White line skipped

Compression argument

This field is reserved. Set the value to 0.

### DEFINE WINDOW PARAMETERS Parameter List for the OneScanner

The window parameter list for the DEFINE WINDOW PARAMETERS command consists of one parameter list header followed by at most one window descriptor. Figure 7-29 shows the format of the parameter list header and descriptor for the OneScanner.

Figure 7-29 shows the DEFINE WINDOW PARAMETERS parameter list for the OneScanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | Data transfer length (MSB) |  |  |  |  |  |  |  |
| 1 | Data transfer length (LSB) |  |  |  |  |  |  |  |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | ($00) Reserved |  |  |  |  |  |  |  |
| 5 | ($00) Reserved |  |  |  |  |  |  |  |
| 6 | Window descriptor length (MSB) |  |  |  |  |  |  |  |
| 7 | Window descriptor length (LSB) |  |  |  |  |  |  |  |

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | Window identifier |  |  |  |  |  |  |  |
| 1 | ($00) Reserved |  |  |  |  |  |  |  |
| 2 - 3 | X-Axis resolution |  |  |  |  |  |  |  |
| 4 - 5 | Y-Axis resolution |  |  |  |  |  |  |  |
| 6 - 9 | X-Axis upper left |  |  |  |  |  |  |  |
| 10 - 13 | Y-Axis upper left |  |  |  |  |  |  |  |
| 14 - 17 | Window width |  |  |  |  |  |  |  |
| 18 - 21 | Window length |  |  |  |  |  |  |  |
| 22 | Brightness |  |  |  |  |  |  |  |
| 23 | Threshold |  |  |  |  |  |  |  |
| 24 | Contrast |  |  |  |  |  |  |  |
| 25 | Image composition |  |  |  |  |  |  |  |
| 26 | Bits per pixel |  |  |  |  |  |  |  |
| 27 - 28 | Halftone pattern |  |  |  |  |  |  |  |
| 29 | ($00) Reserved |  |  |  |  | Padding type |  |  |
| 30 - 31 | Scan direction |  |  |  |  |  |  |  |
| 32 | Compression type |  |  |  |  |  |  |  |
| 33 | Compression argument |  |  |  |  |  |  |  |
| 34 | ($00) Reserved |  |  |  |  |  |  |  |
| 35 | ($00) Reserved |  |  |  |  |  |  |  |
| 36 | ($00) Reserved |  |  |  |  |  |  |  |
| 37 | ($00) Reserved |  |  |  |  |  |  |  |
| 38 | ($00) Reserved |  |  |  |  |  |  |  |
| 39 | ($00) Reserved |  |  |  |  |  |  |  |

FIELD DESCRIPTIONS

The parameter list header describes the length, in bytes, of a window descriptor. An application program may transfer a maximum of one window descriptor. Here are the parameter list fields used in this structure:

Reserved 	These fields are reserved for future expansion. Set them to 0.

Parameter list length

This field specifies the length in bytes of a single window descriptor. Always set this field to 40 ($28).

The window descriptor contains information about one window. Here are the window descriptors used in this structure:

Window identifier

This field identifies the window defined by the descriptor. This field must be set to 0.

Reserved	These fields are reserved for future expansion. Set them to 0.

X resolution	This field specifies the resolution, in dots per inch, along the horizontal direction (x-axis). The OneScanner supports horizontal resolutions that range from 72 to 300 dpi in 1-dpi increments. A value of 0 in this field indicates that the scanner uses a default horizontal resolution value of 72 dpi.

Y resolution This field specifies the resolution, in dots per inch, in the vertical direction (y-axis). The OneScanner supports vertical resolutions that range from 72 to 300 dpi in 1-dpi increments. A value of 0 in this field indicates that the scanner uses a default vertical resolution value of 72 dpi.

Upper-left x This field specifies the x-coordinate of the upper left corner of the rectangular window to be scanned. The point (0,0) is considered the upper-left corner of the window. Coordinates are measured from this point in increments of 1/1200 of an inch from the right edge of the scanner’s glass surface (as viewed when you face the glass). Coordinates must be multiples of 8 times the x resolution; they must lie on byte boundaries. The default value of this field is 0.

Upper-left y This field specifies the y-coordinate of the upper-left corner of the rectangular window to be scanned. The point (0,0) is considered the upper-left corner of the window. Coordinates are measured from this point in increments of 1/1200 of an inch from the top edge of the scanner’s glass surface. The default value of this field is 0.

Width	This field specifies the window width in increments of 1/1200 of an inch along the horizontal axis. This value must be a multiple of 8 times the x resolution; the window border must lie on a byte boundary. The default value of this field is 10,200.

Length	This field specifies the window length in increments of 1/1200 of an inch along the vertical axis. The default value of this field is 13,200.

Brightness	This field specifies the brightness. A value of 0 results in the default value of 128. Any other value indicates a relative brightness: 255 is the highest setting, 1 is the lowest setting, and 128 is the average setting.

Threshold	This field specifies the threshold level. A value of 0 in Line Art mode causes the scanner to use automatic background adjustment. See “MODE SELECT ($15),” earlier in this chapter, for a description of the Auto background-adjustment threshold field. A value of 0 received in any other mode results in the use of the default setting. Any other value indicates a relative threshold parameter: 255 is the highest setting, 1 is the lowest setting, and 128 is the average setting. The default value for this parameter is 128.

Contrast	This field specifies the contrast at which the scanner scans the document. A value of 0 indicates that the scanner uses the default value of 1. Any other value indicates a relative contrast: 255 is the highest setting, 1 is the lowest setting, and 128 is the average setting. The contrast setting is valid only in Halftone mode and Grayscale mode.

Image composition

This field specifies the type of image data acquired. The default composition mode is Line Art mode. The following values are valid:

$00  Line Art mode

$01  Halftone mode

$02  Grayscale mode

Bits per pixel

This field specifies, for any one pixel, the number of bits used to specify the visible density of the document being scanned. This field is valid only if the Image composition field indicates that gray-scale data is desired. The OneScanner supports either 16 or 256 levels of gray in Grayscale mode. For 16-level gray support, set this field to 4; for 256-level gray support, set this field to 8.

Halftone pattern

This field specifies the halftone process by which the scanner converts multilevel gray tones to black and white dots. This parameter is only valid when the Image composition field specifies that halftone image data is desired. The following values are valid:

$00  4-by-4 Spiral

$01  4-by-4 Bayer

$02  Downloaded pattern

$03  8-by-8 Spiral

$04  8-by-8 Bayer

Padding type This field specifies what the scanner does if the image data transmitted to the host is not a whole number of bytes. This field must be $03, which truncates the data to the next byte boundary.

Scan direction

This field specifies the direction for the scan operation. This field must be set to 0, which specifies that the scan is to proceed from left to right and top to bottom.

Compression type

This field specifies the compression technique that the scanner applies to the image data prior to transmission to the host computer. This field must be set to 0, which indicates that the scanner is not to compress the image data.

Compression argument

This field is reserved. Set the value to 0.

### DEFINE WINDOW PARAMETERS Parameter List for the Color OneScanner

The window parameter list for the DEFINE WINDOW PARAMETERS command consists of one parameter list header followed by at most one window descriptor. Figure 7-30 shows the format of the parameter list header and descriptor for the Color OneScanner.

Figure 7-30 shows the DEFINE WINDOW PARAMETERS parameter list for the Color OneScanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | Data transfer length (MSB) |  |  |  |  |  |  |  |
| 1 | Data transfer length (LSB) |  |  |  |  |  |  |  |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | ($00) Reserved |  |  |  |  |  |  |  |
| 5 | ($00) Reserved |  |  |  |  |  |  |  |
| 6 | Window descriptor length (MSB) |  |  |  |  |  |  |  |
| 7 | Window descriptor length (LSB) |  |  |  |  |  |  |  |

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | Window identifier |  |  |  |  |  |  |  |
| 1 | ($00) Reserved |  |  |  |  |  |  |  |
| 2 - 3 | X-Axis resolution |  |  |  |  |  |  |  |
| 4 - 5 | Y-Axis resolution |  |  |  |  |  |  |  |
| 6 - 9 | X-Axis upper left |  |  |  |  |  |  |  |
| 10 - 13 | Y-Axis upper left |  |  |  |  |  |  |  |
| 14 - 17 | Window width |  |  |  |  |  |  |  |
| 18 - 21 | Window length |  |  |  |  |  |  |  |
| 22 | Brightness |  |  |  |  |  |  |  |
| 23 | Threshold |  |  |  |  |  |  |  |
| 24 | Contrast |  |  |  |  |  |  |  |
| 25 | Image composition |  |  |  |  |  |  |  |
| 26 | Bits per pixel |  |  |  |  |  |  |  |
| 27 - 28 | ($00) Halftone pattern |  |  |  |  |  |  |  |
| 29 | ($00) Reserved |  |  |  |  | Padding type |  |  |
| 30 - 31 | ($00) Bit ordering |  |  |  |  |  |  |  |
| 32 | Compression type |  |  |  |  |  |  |  |
| 33 | Compression argument |  |  |  |  |  |  |  |
| 34 | ($00) Reserved |  |  |  |  |  |  |  |
| 35 | ($00) Reserved |  |  |  |  |  |  |  |
| 36 | ($00) Reserved |  |  |  |  |  |  |  |
| 37 | ($00) Reserved |  |  |  |  |  |  |  |
| 38 | ($00) Reserved |  |  |  |  |  |  |  |
| 39 | ($00) Reserved |  |  |  |  |  |  |  |
| 40 | Digital data for VRT |  |  |  |  |  |  |  |
| 41 | Digital data for VRB |  |  |  |  |  |  |  |

FIELD DESCRIPTIONS

The parameter list header describes the length, in bytes, of a window descriptor. An application program may transfer a maximum of one window descriptor. Here are the parameter list fields used in this structure:

Reserved 	These fields are reserved for future expansion. Set them to 0.

Parameter list length

This field specifies the length in bytes of a single window descriptor. Always set this field to 42 ($2A).

The window descriptor contains information about one window. Here are the window descriptors used in this structure:

Window identifier

This field identifies the window defined by the descriptor. This field must be set to 0.

Reserved	These fields are reserved for future expansion. Set them to 0.

X resolution	This field specifies the resolution, in dots per inch, along the horizontal direction (x-axis). The Color OneScanner supports horizontal resolutions that range from 72 to 300 dpi in 1-dpi increments. A value of 0 in this field indicates that the scanner uses a default horizontal resolution value of 72 dpi.

Y resolution This field specifies the resolution, in dots per inch, in the vertical direction (y-axis). The Color OneScanner supports vertical resolutions that range from 72 to 300 dpi in 1-dpi increments. A value of 0 in this field indicates that the scanner uses a default vertical resolution value of 72 dpi.

Upper-left x This field specifies the x-coordinate of the upper left corner of the rectangular window to be scanned. The point (0,0) is considered the upper-left corner of the window. Coordinates are measured from this point in increments of 1/1200 of an inch from the right edge of the scanner’s glass surface (as viewed when you face the glass). Coordinates must be multiples of 8 times the x resolution; they must lie on byte boundaries. The default value of this field is 0.

Upper-left y This field specifies the y-coordinate of the upper-left corner of the rectangular window to be scanned. The point (0,0) is considered the upper-left corner of the window. Coordinates are measured from this point in increments of 1/1200 of an inch from the top edge of the scanner’s glass surface. The default value of this field is 0.

Width	This field specifies the window width in increments of 1/1200 of an inch along the horizontal axis. This value must be a multiple of 8 times the x resolution; the window border must lie on a byte boundary. The default value of this field is 10,200.

Length	This field specifies the window length in increments of 1/1200 of an inch along the vertical axis. The default value of this field is 13,200.

Brightness	This field specifies the brightness. A value of 0 results in the default value of 128. Any other value indicates a relative brightness: 255 is the highest setting, 1 is the lowest setting, and 128 is the average setting.

Threshold	This field specifies the threshold level. A value of 0 in Line Art mode causes the scanner to use automatic background adjustment. See “MODE SELECT ($15),” earlier in this chapter, for a description of the Auto background-adjustment threshold field. A value of 0 received in any other mode results in the use of the default setting. Any other value indicates a relative threshold parameter: 255 is the highest setting, 1 is the lowest setting, and 128 is the average setting. The default value for this parameter is 128.

Contrast	This field specifies the contrast at which the scanner scans the document. A value of 0 indicates that the scanner uses the default value of 1. Any other value indicates a relative contrast: 255 is the highest setting, 1 is the lowest setting, and 128 is the average setting. The contrast setting is valid only in Halftone mode and Grayscale mode.

Image composition

This field specifies the type of image data acquired. The default composition mode is Line Art mode. The following values are valid:

$00  Line Art mode (bi-level black and white)

$02  Grayscale mode (multi-level black and white)

$03  Bi-level Color mode (RGB)

$05  Full Color mode (RGB)

Bits per pixel

This field specifies, for any one pixel, the number of bits used to specify the visible density of the document being scanned. This field is only valid if the Image composition field indicates grayscale data is desired. The Color OneScanner supports 4 ($04) and 8 ($08) bits of gray-scale or color data, 3 bits for bi-level color, and 24 bits for full color (8 bits per color). For 16-level gray support, set this field to 4; for 256-level gray support, set this field to 8.

Halftone pattern

This field is reserved. Set the value to $0000.

Padding type This field specifies what the scanner does if the image data transmitted to the host is not a whole number of bytes. This field must be $03, which truncates the data to the next byte boundary.

Bit ordering	This field is reserved. Set the value to $0000.

Compression type

This field specifies the compression technique that the scanner applies to the image data prior to transmission to the host computer. This field must be set to 0, which indicates that the scanner is not to compress the image data.

Compression argument

This field is reserved. Set the value to 0.

Digital data for Vrt and Vrb

These fields determine the contrast range of a scanned image. Vrt (voltage reference top) determines the upper end of the contrast range, and Vrb (voltage reference bottom) the lower end of the contrast range. If there is a value of 0 in either Vrt or Vrb, Vrt and Vrb are disabled, and Brightness and Contrast are used instead. Any other setting between 1 and 255 indicates the scanner should use the related value, for example, if the setting is 3, the scanner should use the value 3, and so forth.

### DEFINE WINDOW PARAMETERS Parameter List for the Color OneScanner 600/27

The window parameter list for the DEFINE WINDOW PARAMETERS command consists of one parameter list header followed by at most one window descriptor. Figure 7-30B shows the format of the parameter list header and descriptor for the Color OneScanner 600/27.

Figure 7-30B shows the DEFINE WINDOW PARAMETERS parameter list for the Color OneScanner 600/27

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | Data transfer length (MSB) |  |  |  |  |  |  |  |
| 1 | Data transfer length (LSB) |  |  |  |  |  |  |  |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | ($00) Reserved |  |  |  |  |  |  |  |
| 5 | ($00) Reserved |  |  |  |  |  |  |  |
| 6 | Window descriptor length (MSB) |  |  |  |  |  |  |  |
| 7 | Window descriptor length (LSB) |  |  |  |  |  |  |  |

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | Window identifier |  |  |  |  |  |  |  |
| 1 | ($00) Reserved |  |  |  |  |  |  |  |
| 2 - 3 | X-Axis resolution |  |  |  |  |  |  |  |
| 4 - 5 | Y-Axis resolution |  |  |  |  |  |  |  |
| 6 - 9 | X-Axis upper left |  |  |  |  |  |  |  |
| 10 - 13 | Y-Axis upper left |  |  |  |  |  |  |  |
| 14 - 17 | Window width |  |  |  |  |  |  |  |
| 18 - 21 | Window length |  |  |  |  |  |  |  |
| 22 | Brightness |  |  |  |  |  |  |  |
| 23 | Threshold |  |  |  |  |  |  |  |
| 24 | Contrast |  |  |  |  |  |  |  |
| 25 | Image composition |  |  |  |  |  |  |  |
| 26 | Bits per pixel |  |  |  |  |  |  |  |
| 27 - 28 | ($00) Halftone pattern |  |  |  |  |  |  |  |
| 29 | RIF | ($00) Reserved |  |  |  | Padding type |  |  |
| 30 - 31 | ($00) Bit ordering |  |  |  |  |  |  |  |
| 32 | Compression type |  |  |  |  |  |  |  |
| 33 | Compression argument |  |  |  |  |  |  |  |
| 34 | ($00) Reserved |  |  |  |  |  |  |  |
| 35 | ($00) Reserved |  |  |  |  |  |  |  |
| 36 | ($00) Reserved |  |  |  |  |  |  |  |
| 37 | ($00) Reserved |  |  |  |  |  |  |  |
| 38 | ($00) Reserved |  |  |  |  |  |  |  |
| 39 | ($00) Reserved |  |  |  |  |  |  |  |
| 40 | Digital data for VRT |  |  |  |  |  |  |  |
| 41 | Digital data for VRB |  |  |  |  |  |  |  |
| 42 | Canon settings |  |  |  |  |  |  |  |
| 43 | ($00) Reserved |  |  |  |  |  |  |  |
| 44 | ($00) Reserved |  |  |  |  |  |  |  |
| 45 | ($00) Reserved |  |  |  |  |  |  |  |
| 46 | ($02) Reserved |  |  |  |  |  |  |  |
| 47 | ($00) Reserved |  |  |  |  |  |  |  |
| 48 | ($00) Reserved |  |  |  |  |  |  |  |
| 49 | ($01) Reserved |  |  |  |  |  |  |  |
| 50 | ($01) Reserved |  |  |  |  |  |  |  |
| 51 | Hilite level for Red color |  |  |  |  |  |  |  |
| 52 | Shadow level for Red color |  |  |  |  |  |  |  |
| 53 | ($00) Reserved |  |  |  |  |  |  |  |
| 54 | Hilite level for Green color |  |  |  |  |  |  |  |
| 55 | ($00) Reserved |  |  |  |  |  |  |  |
| 56 | Shadow level for Green colur |  |  |  |  |  |  |  |
| 57 | ($00) Reserved |  |  |  |  |  |  |  |
| 58 | ($00) Reserved |  |  |  |  |  |  |  |
| 59 | ($00) Reserved |  |  |  |  |  |  |  |
| 60 | ($00) Reserved |  |  |  |  |  |  |  |
| 61 | ($00) Reserved |  |  |  |  |  |  |  |
| 62 | Hilite level for Blue color |  |  |  |  |  |  |  |
| 63 | Shadow level for Blue color |  |  |  |  |  |  |  |

FIELD DESCRIPTIONS

The parameter list header describes the length, in bytes, of a window descriptor. An application program may transfer a maximum of one window descriptor. Here are the parameter list fields used in this structure:

Reserved 	These fields are reserved for future expansion. Set them to 0.

Parameter list length

This field specifies the length in bytes of a single window descriptor. Always set this field to 64 ($40).

The window descriptor contains information about one window. Here are the window descriptors used in this structure:

Window identifier

This field identifies the window defined by the descriptor. This field must be set to 0.

Reserved	These fields are reserved for future expansion. Set them to 0.

X resolution	This field specifies the resolution, in dots per inch, along the horizontal direction (x-axis). The Color OneScanner 600/27 supports horizontal resolutions that range from 72 to 300 dpi in 1-dpi increments. A value of 0 in this field indicates that the scanner uses a default horizontal resolution value of 72 dpi.

Y resolution This field specifies the resolution, in dots per inch, in the vertical direction (y-axis). The Color OneScanner 600/27 supports vertical resolutions that range from 72 to 600 dpi in 1-dpi increments. A value of 0 in this field indicates that the scanner uses a default vertical resolution value of 72 dpi.

Upper-left x This field specifies the x-coordinate of the upper left corner of the rectangular window to be scanned. The point (0,0) is considered the upper-left corner of the window. Coordinates are measured from this point in increments of 1/1200 of an inch from the right edge of the scanner’s glass surface (as viewed when you face the glass). Coordinates must be multiples of 8 times the x resolution; they must lie on byte boundaries. The default value of this field is 0.

Upper-left y This field specifies the y-coordinate of the upper-left corner of the rectangular window to be scanned. The point (0,0) is considered the upper-left corner of the window. Coordinates are measured from this point in increments of 1/1200 of an inch from the top edge of the scanner’s glass surface. The default value of this field is 0.

Width	This field specifies the window width in increments of 1/1200 of an inch along the horizontal axis. This value must be a multiple of 8 times the x resolution; the window border must lie on a byte boundary. The default value of this field is 10,200.

Length	This field specifies the window length in increments of 1/1200 of an inch along the vertical axis. The default value of this field is 13,200.

Brightness	This field specifies the brightness. A value of 0 results in the default value of 128. Any other value indicates a relative brightness: 255 is the highest setting, 1 is the lowest setting, and 128 is the average setting.

Threshold	This field specifies the threshold level. A value of 0 in Line Art mode causes the scanner to use automatic background adjustment. See “MODE SELECT ($15),” earlier in this chapter, for a description of the Auto background-adjustment threshold field. A value of 0 received in any other mode results in the use of the default setting. Any other value indicates a relative threshold parameter: 255 is the highest setting, 1 is the lowest setting, and 128 is the average setting. The default value for this parameter is 128.

Contrast	This field specifies the contrast at which the scanner scans the document. A value of 0 indicates that the scanner uses the default value of 1. Any other value indicates a relative contrast: 255 is the highest setting, 1 is the lowest setting, and 128 is the average setting. The contrast setting is valid only in Halftone mode and Grayscale mode.

Image composition

This field specifies the type of image data acquired. The default composition mode is Line Art mode. The following values are valid:

$00  Line Art mode (bi-level black and white)

$02  Grayscale mode (multi-level black and white)

$03  Bi-level Color mode (RGB)

$05  Full Color mode (RGB)

Bits per pixel

This field specifies, for any one pixel, the number of bits used to specify the visible density of the document being scanned. This field is only valid if the Image composition field indicates grayscale data is desired. The Color OneScanner 600/27 supports 4 ($04) and 8 ($08) bits of gray-scale or color data, 3 bits for bi-level color, and 24 bits for full color (8 bits per color). For 16-level gray support, set this field to 4; for 256-level gray support, set this field to 8.

Halftone pattern

This field is reserved. Set the value to $0000.

Reverse Image Format (RIF)

This field specifies if the source document is a negative film or a normal document. Set the bit to 0 if the source document is a negative film, or set the bit to 1 if the source document is a normal document

Padding type This field specifies what the scanner does if the image data transmitted to the host is not a whole number of bytes. This field must be $03, which truncates the data to the next byte boundary.

Bit ordering	This field is reserved. Set the value to $0000.

Compression type

This field specifies the compression technique that the scanner applies to the image data prior to transmission to the host computer. This field must be set to 0, which indicates that the scanner is not to compress the image data.

Compression argument

This field is reserved. Set the value to 0.

Digital data for Vrt and Vrb

These fields determine the contrast range of a scanned image. Vrt (voltage reference top) determines the upper end of the contrast range, and Vrb (voltage reference bottom) the lower end of the contrast range. If there is a value of 0 in either Vrt or Vrb, Vrt and Vrb are disabled, and Brightness and Contrast are used instead. Any other setting between 1 and 255 indicates the scanner should use the related value, for example, if the setting is 3, the scanner should use the value 3, and so forth.

Canon settings

This field specifies some Canon-specific parameters for scanning.

Gray Response Curve, bit 3 - Set to 0 if you do not activate the gray response curve. Set to 1 if you activate the gray response curve.

Mirror, bit 2 - Set to 0 if the image is returned as it is scanned. Set to 1 if the returned image will be returned mirrored.

Auto-Exposure, bit 0 - Set to 0 if you do not acivate the auto-exposure setting. Set to 1 if you activate the auto-exposure setting.

Highlight and Shadow levels

Highlights are the lightest elements in an image, whereas shadows are the darkest areas. You can set a highlight and a shadow value in the range from 0 to 255 for each of the three components R, G, and B.

### DEFINE WINDOW PARAMETERS Parameter List for the Color OneScanner 1200/30

The window parameter list for the DEFINE WINDOW PARAMETERS command consists of one parameter list header followed by at most one window descriptor. Figure 7-30B shows the format of the parameter list header and descriptor for the Color OneScanner 1200/30.

Figure 7-30C shows the DEFINE WINDOW PARAMETERS parameter list for the Color OneScanner 1200/30

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | Data transfer length (MSB) |  |  |  |  |  |  |  |
| 1 | Data transfer length (LSB) |  |  |  |  |  |  |  |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | ($00) Reserved |  |  |  |  |  |  |  |
| 5 | ($00) Reserved |  |  |  |  |  |  |  |
| 6 | Window descriptor length (MSB) |  |  |  |  |  |  |  |
| 7 | Window descriptor length (LSB) |  |  |  |  |  |  |  |

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | Window identifier |  |  |  |  |  |  |  |
| 1 | ($00) Reserved |  |  |  |  |  |  |  |
| 2 - 3 | X-Axis resolution |  |  |  |  |  |  |  |
| 4 - 5 | Y-Axis resolution |  |  |  |  |  |  |  |
| 6 - 9 | X-Axis upper left |  |  |  |  |  |  |  |
| 10 - 13 | Y-Axis upper left |  |  |  |  |  |  |  |
| 14 - 17 | Window width |  |  |  |  |  |  |  |
| 18 - 21 | Window length |  |  |  |  |  |  |  |
| 22 | Brightness |  |  |  |  |  |  |  |
| 23 | Threshold |  |  |  |  |  |  |  |
| 24 | Contrast |  |  |  |  |  |  |  |
| 25 | Image composition |  |  |  |  |  |  |  |
| 26 | Bits per pixel |  |  |  |  |  |  |  |
| 27 - 28 | ($00) Halftone pattern |  |  |  |  |  |  |  |
| 29 | RIF | ($00) Reserved |  |  |  | Padding type |  |  |
| 30 - 31 | ($00) Bit ordering |  |  |  |  |  |  |  |
| 32 | Compression type |  |  |  |  |  |  |  |
| 33 | Compression argument |  |  |  |  |  |  |  |
| 34 | ($00) Reserved |  |  |  |  |  |  |  |
| 35 | ($00) Reserved |  |  |  |  |  |  |  |
| 36 | ($00) Reserved |  |  |  |  |  |  |  |
| 37 | ($00) Reserved |  |  |  |  |  |  |  |
| 38 | ($00) Reserved |  |  |  |  |  |  |  |
| 39 | ($00) Reserved |  |  |  |  |  |  |  |
| 40 | Digital data for VRT |  |  |  |  |  |  |  |
| 41 | Digital data for VRB |  |  |  |  |  |  |  |
| 42 | Canon settings |  |  |  |  |  |  |  |
| 43 | ($00) Reserved |  |  |  |  |  |  |  |
| 44 | ($00) Reserved |  |  |  |  |  |  |  |
| 45 | ($00) Reserved |  |  |  |  |  |  |  |
| 46 | ($02) Reserved |  |  |  |  |  |  |  |
| 47 | ($00) Reserved |  |  |  |  |  |  |  |
| 48 | ($00) Reserved |  |  |  |  |  |  |  |
| 49 | ($01) Reserved |  |  |  |  |  |  |  |
| 50 | ($01) Reserved |  |  |  |  |  |  |  |
| 51 | Hilite level for Red color |  |  |  |  |  |  |  |
| 52 | Shadow level for Red color |  |  |  |  |  |  |  |
| 53 | ($00) Reserved |  |  |  |  |  |  |  |
| 54 | Hilite level for Green color |  |  |  |  |  |  |  |
| 55 | ($00) Reserved |  |  |  |  |  |  |  |
| 56 | Shadow level for Green colur |  |  |  |  |  |  |  |
| 57 | ($00) Reserved |  |  |  |  |  |  |  |
| 58 | ($00) Reserved |  |  |  |  |  |  |  |
| 59 | ($00) Reserved |  |  |  |  |  |  |  |
| 60 | ($00) Reserved |  |  |  |  |  |  |  |
| 61 | ($00) Reserved |  |  |  |  |  |  |  |
| 62 | Hilite level for Blue color |  |  |  |  |  |  |  |
| 63 | Shadow level for Blue color |  |  |  |  |  |  |  |

FIELD DESCRIPTIONS

The parameter list header describes the length, in bytes, of a window descriptor. An application program may transfer a maximum of one window descriptor. Here are the parameter list fields used in this structure:

Reserved 	These fields are reserved for future expansion. Set them to 0.

Parameter list length

This field specifies the length in bytes of a single window descriptor. Always set this field to 64 ($40).

The window descriptor contains information about one window. Here are the window descriptors used in this structure:

Window identifier

This field identifies the window defined by the descriptor. This field must be set to 0.

Reserved	These fields are reserved for future expansion. Set them to 0.

X resolution	This field specifies the resolution, in dots per inch, along the horizontal direction (x-axis). The Color OneScanner 1200/30 supports horizontal resolutions that range from 72 to 600 dpi in 1-dpi increments. A value of 0 in this field indicates that the scanner uses a default horizontal resolution value of 72 dpi.

Y resolution This field specifies the resolution, in dots per inch, in the vertical direction (y-axis). The Color OneScanner 1200/30 supports vertical resolutions that range from 72 to 1200 dpi in 1-dpi increments. A value of 0 in this field indicates that the scanner uses a default vertical resolution value of 72 dpi.

Upper-left x This field specifies the x-coordinate of the upper left corner of the rectangular window to be scanned. The point (0,0) is considered the upper-left corner of the window. Coordinates are measured from this point in increments of 1/1200 of an inch from the right edge of the scanner’s glass surface (as viewed when you face the glass). Coordinates must be multiples of 8 times the x resolution; they must lie on byte boundaries. The default value of this field is 0.

Upper-left y This field specifies the y-coordinate of the upper-left corner of the rectangular window to be scanned. The point (0,0) is considered the upper-left corner of the window. Coordinates are measured from this point in increments of 1/1200 of an inch from the top edge of the scanner’s glass surface. The default value of this field is 0.

Width	This field specifies the window width in increments of 1/1200 of an inch along the horizontal axis. This value must be a multiple of 8 times the x resolution; the window border must lie on a byte boundary. The default value of this field is 10,200.

Length	This field specifies the window length in increments of 1/1200 of an inch along the vertical axis. The default value of this field is 13,200.

Brightness	This field specifies the brightness. A value of 0 results in the default value of 128. Any other value indicates a relative brightness: 255 is the highest setting, 1 is the lowest setting, and 128 is the average setting.

Threshold	This field specifies the threshold level. A value of 0 in Line Art mode causes the scanner to use automatic background adjustment. See “MODE SELECT ($15),” earlier in this chapter, for a description of the Auto background-adjustment threshold field. A value of 0 received in any other mode results in the use of the default setting. Any other value indicates a relative threshold parameter: 255 is the highest setting, 1 is the lowest setting, and 128 is the average setting. The default value for this parameter is 128.

Contrast	This field specifies the contrast at which the scanner scans the document. A value of 0 indicates that the scanner uses the default value of 1. Any other value indicates a relative contrast: 255 is the highest setting, 1 is the lowest setting, and 128 is the average setting. The contrast setting is valid only in Halftone mode and Grayscale mode.

Image composition

This field specifies the type of image data acquired. The default composition mode is Line Art mode. The following values are valid:

$00  Line Art mode (bi-level black and white)

$02  Grayscale mode (multi-level black and white)

$03  Bi-level Color mode (RGB)

$05  Full Color mode (RGB)

Bits per pixel

This field specifies, for any one pixel, the number of bits used to specify the visible density of the document being scanned. This field is only valid if the Image composition field indicates grayscale data is desired. The Color OneScanner 1200/30 supports 4 ($04) and 8 ($08) bits of gray-scale or color data, 3 bits for bi-level color, and 24 bits for full color (8 bits per color). For 16-level gray support, set this field to 4; for 256-level gray support, set this field to 8.

Halftone pattern

This field is reserved. Set the value to $0000.

Reverse Image Format (RIF)

This field specifies if the source document is a negative film or a normal document. Set the bit to 0 if the source document is a negative film, or set the bit to 1 if the source document is a normal document

Padding type This field specifies what the scanner does if the image data transmitted to the host is not a whole number of bytes. This field must be $03, which truncates the data to the next byte boundary.

Bit ordering	This field is reserved. Set the value to $0000.

Compression type

This field specifies the compression technique that the scanner applies to the image data prior to transmission to the host computer. This field must be set to 0, which indicates that the scanner is not to compress the image data.

Compression argument

This field is reserved. Set the value to 0.

Digital data for Vrt and Vrb

These fields determine the contrast range of a scanned image. Vrt (voltage reference top) determines the upper end of the contrast range, and Vrb (voltage reference bottom) the lower end of the contrast range. If there is a value of 0 in either Vrt or Vrb, Vrt and Vrb are disabled, and Brightness and Contrast are used instead. Any other setting between 1 and 255 indicates the scanner should use the related value, for example, if the setting is 3, the scanner should use the value 3, and so forth.

Canon settings

This field specifies some Canon-specific parameters for scanning.

Gray Response Curve, bit 3 - Set to 0 if you do not activate the gray response curve. Set to 1 if you activate the gray response curve.

Mirror, bit 2 - Set to 0 if the image is returned as it is scanned. Set to 1 if the returned image will be returned mirrored.

Auto-Exposure, bit 0 - Set to 0 if you do not acivate the auto-exposure setting. Set to 1 if you activate the auto-exposure setting.

Highlight and Shadow levels

Highlights are the lightest elements in an image, whereas shadows are the darkest areas. You can set a highlight and a shadow value in the range from 0 to 255 for each of the three components R, G, and B.

### Using Window Descriptors With the Apple Scanner

Note

This section applies only to the Apple Scanner.

If the Apple bit in a DEFINE WINDOW PARAMETERS command structure is set to 1, an application program may send more than one window descriptor. The first window descriptor defines the primary window as well as the parameters for the entire scan. The additional window descriptors define secondary windows within the primary window, each of which may have a different image composition type. For example, the primary window could be used to define a scan as Line Art mode, and you may also define secondary windows within the primary window as Halftone composition.

Since halftone data is basically just a black-and-white image, the halftone data and line art data can be mixed freely. However, only halftone data and line art data can be mixed. Mixing with grayscale data is not allowed. If windows overlap, the window descriptor with the higher window identifier value takes priority.

Additional window descriptors must have all fields set to the same value as those in the primary window descriptor, except for the five fields that define the window rectangle (upper-left y, upper-left x, Width, Length, and Window identifier) and the Composition field. Window descriptor fields with different values result in an error. The byte- boundary restriction that applies to the parameters Width and upper-left x of the primary window does not apply to those parameters for secondary windows.

You may specify any window identifier as a parameter in the DEFINE WINDOW PARAMETERS command. All other commands—such as SCAN, GET DATA BUFFER STATUS, and READ—accept only the window identifier of a primary window.

### Structure details

#### Image composition codes

| Code | Description |
| --- | --- |
| 00h | Bi-level black & white |
| 01h | Dithered/halftone black & white |
| 02h | Multi-level black & white (gray scale) |
| 03h | Bi-level RGB colour |
| 04h | Dithered/halftone RGB colour |
| 05h | Multi-level RGB colour |
| 06h - FFh | Reserved |

#### Padding types

| Code | Description |
| --- | --- |
| 00h | No padding |
| 01h | Pad with 0's to byte boundary |
| 02h | Pad with 1's to byte boundary |
| 03h | Truncate to byte boundary |
| 04h - FFh | Reserved |

#### Compression types and arguments

| Code | Description | Compression argument |
| --- | --- | --- |
| 00h | No compression | Reserved |
| 01h | CCITT group III, 1 dimensional | Reserved |
| 02h | CCITT group III, 2 dimensional | K factor |
| 03h | CCITT group IV, 2 dimensional | Reserved |
| 04h - 0Fh | Reserved | Reserved |
| 10h | Optical character recognition (OCR) | Vendor-specific |
| 11h - 7Fh | Reserved | Reserved |
| 80h - FFh | Vendor-specific | Vendor-specific |

## GET WINDOW PARAMETERS ($25)

The GET WINDOWS PARAMETERS command provides a means for the initiator to get information about a window previously defined using the DEFINE WINDOW PARAMETERS command. Figure 7-31 shows the format of the GET WINDOW PARAMETERS command structure.

Note

This command is supported only by the Color OneScanner.

Figure 7-31 The GET WINDOW PARAMETERS command structure

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($25) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) Reserved |  |  |  | Single |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | ($00) Reserved |  |  |  |  |  |  |  |
| 5 | ($00) Window identifier |  |  |  |  |  |  |  |
| 6 | Transfer length (MSB) |  |  |  |  |  |  |  |
| 7 | Transfer length |  |  |  |  |  |  |  |
| 8 | Transfer length (LSB) |  |  |  |  |  |  |  |
| 9 | Control |  |  |  |  |  |  |  |

FIELD DESCRIPTIONS

Here is a list of the fields and bits used in this command structure:

Operation code

The operation code for the GET WINDOW PARAMETERS command is $25.

Reserved 	These fields are reserved for future expansion. Set them to 0.

Single	A single bit of one specifies that a single window descriptor shall be returned for the specified window identifier. A single bit of zero specifies that window descriptors be returned for all window identifiers that were defined by a SET WINDOWS command or by the target, if the automatic bit was set to one.

Window identifier

The only valid value for this field is $00.

Transfer length

These fields specify the length, in bytes, of the window description information sent to the initiator. If the value of the transfer length is 0, no window description information was sent. This is not considered an error. The scanner stops transferring data when it has returned data equivalent to the transfer byte length, or when it has returned all appropriate GET WINDOWS data, whichever comes first.

Figure 7-32 shows the GET WINDOW PARAMETERS parameter list.

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | Data transfer length (MSB) |  |  |  |  |  |  |  |
| 1 | Data transfer length (LSB) |  |  |  |  |  |  |  |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | ($00) Reserved |  |  |  |  |  |  |  |
| 5 | ($00) Reserved |  |  |  |  |  |  |  |
| 6 | ($00) Window descriptor length (MSB) |  |  |  |  |  |  |  |
| 7 | ($2A) Window descriptor length (LSB) |  |  |  |  |  |  |  |

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | Window identifier |  |  |  |  |  |  |  |
| 1 | ($00) Reserved |  |  |  |  |  |  |  |
| 2 - 3 | X-Axis resolution |  |  |  |  |  |  |  |
| 4 - 5 | Y-Axis resolution |  |  |  |  |  |  |  |
| 6 - 9 | X-Axis upper left |  |  |  |  |  |  |  |
| 10 - 13 | Y-Axis upper left |  |  |  |  |  |  |  |
| 14 - 17 | Window width |  |  |  |  |  |  |  |
| 18 - 21 | Window length |  |  |  |  |  |  |  |
| 22 | Brightness |  |  |  |  |  |  |  |
| 23 | Threshold |  |  |  |  |  |  |  |
| 24 | Contrast |  |  |  |  |  |  |  |
| 25 | Image composition |  |  |  |  |  |  |  |
| 26 | Bits per pixel |  |  |  |  |  |  |  |
| 27 - 28 | Halftone pattern |  |  |  |  |  |  |  |
| 29 | RIF | ($00) Reserved |  |  |  | Padding type |  |  |
| 30 - 31 | Bit ordering |  |  |  |  |  |  |  |
| 32 | Compression type |  |  |  |  |  |  |  |
| 33 | Compression argument |  |  |  |  |  |  |  |
| 34 | ($00) Reserved |  |  |  |  |  |  |  |
| 35 | ($00) Reserved |  |  |  |  |  |  |  |
| 36 | ($00) Reserved |  |  |  |  |  |  |  |
| 37 | ($00) Reserved |  |  |  |  |  |  |  |
| 38 | ($00) Reserved |  |  |  |  |  |  |  |
| 39 | ($00) Reserved |  |  |  |  |  |  |  |
| 40 | DD (Digital data) for VRT |  |  |  |  |  |  |  |
| 41 | DD (Digital data) for VRB |  |  |  |  |  |  |  |

Refer to the DEFINE WINDOW PARAMETERS section above for detailed information of each field.

## READ ($28)

The READ command instructs the scanner to send data currently in its memory to the host computer. With the Apple Scanner, the READ command may return only image data. With the OneScanner, the READ command may return image data or the contents of the scanner’s calibration RAM or static RAM. The Color OneScanner may return image data, the 3-by-3 matrix for color correction, the gamma function table, and PSRAM data.

Use the GET DATA STATUS command to determine the amount of image data available for a particular window. See “GET DATA STATUS ($34),” later in this chapter, for more information.

### READ Command Structure for the Apple Scanner

Figure 7-33 shows the format of the READ command structure for the Apple Scanner.

Figure 7-33 The READ command structure for the Apple Scanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($28) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) Reserved |  |  |  |  |
| 2 | ($00) Data type code |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | Data type qualifier (MSB) |  |  |  |  |  |  |  |
| 5 | Data type qualifier (LSB) |  |  |  |  |  |  |  |
| 6 | Transfer length (MSB) |  |  |  |  |  |  |  |
| 7 | Transfer length |  |  |  |  |  |  |  |
| 8 | Transfer length (LSB) |  |  |  |  |  |  |  |
| 9 | Control |  |  |  |  |  |  |  |

FIELD DESCRIPTIONS

Here is a list of the fields used in this command structure:

Operation code

The operation code for the READ command is $28.

Reserved 	These fields are reserved for future expansion. Set them to 0.

Data type code

This field specifies the type of data to be read. It must be set to $00.

Data type qualifier

This field specifies the window for an image data read operation. Before the scanner can read data from a window, your application program must define the window in a DEFINE WINDOW PARAMETERS command and then include it in the window list of a SCAN command.

Transfer length

This field specifies the maximum number of bytes that the host computer has allocated for the returned data. If the allocation length is 0, the scanner transfers no data (this is not an error condition). The command is terminated either when the host computer receives Allocation length bytes or when the scanner sends all available data, whichever is less. The GET DATA STATUS command determines the amount of data available for a given window.

### READ Command Structure for the OneScanner

Figure 7-34 shows the READ command structure for the OneScanner. When reading the OneScanner calibration RAM, you must issue a read request for 2,550 bytes. When reading a downloaded halftone pattern from the OneScanner static RAM, you may read all or part of the pattern.

Figure 7-34 The READ command structure for the OneScanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($28) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) Reserved |  |  |  |  |
| 2 | ($00 or $02 or $80) Data type code |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | ($00) Data type qualifier (MSB) |  |  |  |  |  |  |  |
| 5 | ($00) Data type qualifier (LSB) |  |  |  |  |  |  |  |
| 6 | Transfer length (MSB) |  |  |  |  |  |  |  |
| 7 | Transfer length |  |  |  |  |  |  |  |
| 8 | Transfer length (LSB) |  |  |  |  |  |  |  |
| 9 | Control |  |  |  |  |  |  |  |

FIELD DESCRIPTIONS

Here is a list of the fields used in this command structure:

Operation code

The operation code for the READ command is $28.

Reserved 	These fields are reserved for future expansion. Set them to 0.

Data type code

This field specifies the type of data to be read. Set this field to one of the following values:

$00 Read image data

$02 Read halftone pattern from static RAM

$80 Read calibration RAM

Data type qualifier

This field specifies the window for an image data read operation. Before the scanner can read data from a window, your application program must define the window in a DEFINE WINDOW PARAMETERS command and then include it in the window list of a SCAN command.

Transfer length

This field specifies the maximum number of bytes that the host computer has allocated for the returned data. If the allocation length is 0, the scanner transfers no data (this is not an error condition). The command is terminated either when the host computer receives Allocation length bytes or when the scanner sends all available data, whichever is less. The GET DATA STATUS command determines the amount of data available for a given window.

When reading the contents of the processor static RAM (Transfer data type set to $02), you may read some or all of a downloaded halftone pattern. Set Allocation length to a value less than or equal to the size of the stored halftone matrix plus the dimension byte (see “Halftone Parameter Page Description,” later in this chapter, for a detailed description of the format of a halftone matrix). You may not read the contents of any of the built-in halftone patterns.

When reading the contents of calibration RAM (Transfer data type set to $80), you may read some or all of that memory. Set Allocation length to a value in the range from 1 through 255. Each returned byte contains the calibration data for the corresponding pixel sensor in the CCD array.

### READ Command Structure for the Color OneScanner

Figure 7-35 shows the READ command structure for the Color OneScanner. When reading the Color OneScanner calibration RAM, you must issue a read request for 8,100 bytes. When reading the 3-by-3 matrix table, you must issue a read request for 18 bytes. When reading the gamma table, you must issue a read request for 768 bytes.

Figure 7-35 The READ command structure for the Color OneScanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($28) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) Reserved |  |  |  |  |
| 2 | ($00 or $01 or $03 or $80) Data type code |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | ($00) Data type qualifier (MSB) |  |  |  |  |  |  |  |
| 5 | ($00 or $02) Data type qualifier (LSB) |  |  |  |  |  |  |  |
| 6 | Transfer length (MSB) |  |  |  |  |  |  |  |
| 7 | Transfer length |  |  |  |  |  |  |  |
| 8 | Transfer length (LSB) |  |  |  |  |  |  |  |
| 9 | Control |  |  |  |  |  |  |  |

FIELD DESCRIPTIONS

Here is a list of the fields used in this command structure:

Operation code

The operation code for the READ command is $28.

Reserved 	These fields are reserved for future expansion. Set them to 0.

Data type code

This field specifies the type of data to be read. Set this field to one of the following values:

$00 Read image data

$01 Read 3-by-3 matrix for color correction

$03 Read gamma function table

$80 Read calibration RAM

Data type qualifier

This field specifies the window for an image data read operation. Before the scanner can read data from a window, your application program must define the window in a DEFINE WINDOW PARAMETERS command and then include it in the window list of a SCAN command. The first byte of the Transfer ID field is set to $00, meaning only one window is supported. The second byte of the Transfer ID field differentiates between data transfers of the same Transfer data type, as shown in Table 7-2.

Transfer length

This field specifies the maximum number of bytes that the host computer has allocated for the returned data. If the allocation length is 0, the scanner transfers no data (this is not an error condition). The command is terminated either when the host computer receives Allocation length bytes or when the scanner sends all available data, whichever is less. The GET DATA STATUS command determines the amount of data available for a given window.

When reading the contents of the calibration RAM (Transfer data type set to $80), you may read some or all of that memory. Set Allocation length to 8100. Each returned byte contains the calibration data for the correspond- ing pixel sensor in the CCD array. When reading the 3-by-3 matrix table, set the Allocation length to 18 bytes. When reading the Gamma table, set the Allocation length to 768 bytes

When reading image data, the data transferred must always be an even number of bytes. If the initiator asks for an odd number of bytes, the scanner returns a CHECK CONDITION status. If the initiator then sends a REQUEST SENSE command, the scanner returns SENSE KEY $5, indicating that this is an illegal transaction. If the initiator sends SCSI commands REQUEST SENSE, INQUIRY, or GET WINDOW PARAMETERS, it can then ask for either an odd or an even number of bytes of data.

Table 7-2 Transfer data types and transfer identification

| Data type code | Data type qualifier | Description |
| --- | --- | --- |
| $00 | $00 | Image data |
| $01 | $02 | 3-by-3 color correction matrix |
| $03 | $02 | Gamma function table |
| $80 | $02 | PSRAM table for red/green/blue |

The internal hardware structure of the scanner requires that each color of each line, or each line, in the case of monochrome modes, is composed of an even number of bytes. If the scan area requested results in a line that is not an even number of bytes in length, the scanner increases the scan width so that the scan line is an even number of bytes. Figure 7-36 illustrates what happens in this case. The host then adds the additional number of bytes, color, and lines to the originally requested scan width when reading the image data.

Figure 7-36 Extra bit or byte returned for different resolutions and composition modes

| Number of pixels per line | Image composition monochrome | Additional bit or byte per line | Image composition color | Additional bit or byte |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | R | G | B | Total |
| 16N | 1 bit | 0 bits | 3 bit | 0 bits | 0 bits | 0 bits | 0 bits |
| 16N+1 |  | 15 bits (15 pixels) |  | 15 bits | 15 bits | 15 bits | 45 bytes (15 pixels) |
| 16N+2 |  | 14 bits (14 pixels) |  | 14 bits | 14 bits | 14 bits | 42 bits (14 pixels) |
| 16N+3 |  | 13 bits (13 pixels) |  | 13 bits | 13 bits | 13 bits | 39 bits (13 pixels) |
| 16N+4 |  | 12 bits (12 pixels) |  | 12 bits | 12 bits | 12 bits | 36 bits (12 pixels) |
| 16N+5 |  | 11 bits (11 pixels) |  | 11 bits | 11 bits | 11 bits | 33 bits (11 pixels) |
| 16N+6 |  | 10 bits (10 pixels) |  | 10 bits | 10 bits | 10 bits | 30 bits (10 pixels) |
| 16N+7 |  | 9 bits (9 pixels) |  | 9 bits | 9 bits | 9 bits | 27 bits (9 pixels) |
| 16N+8 |  | 8 bits (8 pixels) |  | 8 bits | 8 bits | 8 bits | 24 bits (8 pixels) |
| 16N+9 |  | 7 bits (7 pixels) |  | 7 bits | 7 bits | 7 bits | 21 bits (7 pixels) |
| 16N+10 |  | 6 bits (6 pixels) |  | 6 bits | 6 bits | 6 bits | 18 bits (6 pixels) |
| 16N+11 |  | 5 bits (5 pixels) |  | 5 bits | 5 bits | 5 bits | 15 bits (5 pixels) |
| 16N+12 |  | 4 bits (4 pixels) |  | 4 bits | 4 bits | 4 bits | 12 bits (4 pixels) |
| 16N+13 |  | 3 bits (3 pixels) |  | 3 bits | 3 bits | 3 bits | 9 bits (3 pixels) |
| 16N+14 |  | 2 bits (2 pixels) |  | 2 bits | 2 bits | 2 bits | 6 bits (2 pixels) |
| 16N+15 |  | 1 bits (1 pixels) |  | 1 bits | 1 bits | 1 bits | 3 bits (1 pixel) |
| 4N | 4 bits | 0 bits |  |  |  |  |  |
| 4N+1 |  | 12 bits (3 pixels) |  |  |  |  |  |
| 4N+2 |  | 8 bits (2 pixels) |  |  |  |  |  |
| 4N+3 |  | 4 bits (1 pixels) |  |  |  |  |  |
| 8N | 8 bits | 0 bytes | 24 bits | 0 bytes | 0 bytes | 0 bytes | 0 bytes |
| 8B+1 |  |  |  | 1 byte | 1 byte | 1 byte | 3 bytes (1 pixel) |

### Structure details

#### Data type codes

| Code | Description |
| --- | --- |
| 00h | Image |
| 01h | Vendor-specific |
| 02h | Halftone mask |
| 03h | Gamma function |
| 04h - 7Fh | Reserved |
| 80h - FFh | Vendor-specific |

## SEND ($2A)

The SEND command provides a means for the scanner to receive parameter data from the host computer. With the Apple Scanner, you can use this command to download halftone matrixes. With the OneScanner, you can use this command to download halftone matrixes or to set the contents of the scanner’s calibration RAM. The Color OneScanner downloads a 3-by-3 matrix for color correction, the gamma table, and PSRAM data write. This section describes the SEND command structures for the Apple Scanner, the Apple OneScanner, and the Apple Color OneScanner. It also details the halftone matrix format for the Scanner and the OneScanner, and the Color OneScanner’s 3-by-3 matrix, gamma table, and PSRAM data write.

### SEND Command Structure for the Apple Scanner

Figure 7-37 shows the format of the SEND command structure for the Apple Scanner.

Figure 7-37 The SEND command structure for the Apple Scanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($2A) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) Reserved |  |  |  |  |
| 2 | ($02) Data type code |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | ($00) Data type qualifier (MSB) |  |  |  |  |  |  |  |
| 5 | ($02) Data type qualifier (LSB) |  |  |  |  |  |  |  |
| 6 | Transfer length (MSB) |  |  |  |  |  |  |  |
| 7 | Transfer length |  |  |  |  |  |  |  |
| 8 | ($11) Transfer length (LSB) |  |  |  |  |  |  |  |
| 9 | Control |  |  |  |  |  |  |  |

FIELD DESCRIPTIONS

Here is a list of the fields used in this command structure:

Operation code

The operation code for the SEND command is $2A.

Reserved 	These fields are reserved for future expansion. Set them to 0.

Data type code

This field indicates the type of parameter data to be transferred to the scanner. Only halftone parameter data may be transferred to the Apple Scanner. Set this field to $02.

Data type qualifer

This field indicates which halftone matrix within the scanner will be altered. This field must always be $02.

Transfer length

This field indicates the amount of parameter data to be sent. Since the Apple Scanner supports this command only for 4-by-4 halftone matrixes, this parameter must always be set to 17 ($11) — the size of a 4-by-4 halftone parameter page.

### SEND Command Structure for the OneScanner

Figure 7-38 shows the format of the SEND command structure for the OneScanner.

Figure 7-38 The SEND command structure for the OneScanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($2A) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) Reserved |  |  |  |  |
| 2 | ($02 or $80) Data type code |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | Data type qualifier (MSB) |  |  |  |  |  |  |  |
| 5 | Data type qualifier (LSB) |  |  |  |  |  |  |  |
| 6 | Transfer length (MSB) |  |  |  |  |  |  |  |
| 7 | Transfer length |  |  |  |  |  |  |  |
| 8 | Transfer length (LSB) |  |  |  |  |  |  |  |
| 9 | Control |  |  |  |  |  |  |  |

FIELD DESCRIPTIONS

Here is a list of the fields used in this command structure:

Operation code

The operation code for the SEND command is $2A.

Reserved 	These fields are reserved for future expansion. Set them to 0.

Data type code

This field indicates the type of parameter data to be transferred to the scanner. Set this field to one of the following values:

$02 Send halftone pattern to static RAM

$80 Set calibration RAM

Data type qualifer

This field indicates which halftone matrix within the scanner will be altered. This field must always be $02.

Transfer length

This field indicates the amount of parameter data to be sent. For halftone matrixes, set this field to the size of the matrix plus the dimension byte. (See “Halftone Parameter Page Description,” later in this chapter, for a detailed description of the format of a halftone matrix.) The OneScanner supports both 4-by-4 and 8-by-8 halftone matrixes.

When setting the contents of the calibration RAM, you must set Transfer length to 2550. The parameter data consists of a contiguous buffer of 2550 bytes. Each byte of parameter data contains the new calibration data for the corresponding pixel sensor in the CCD array.

### SEND Command Structure for the Color OneScanner

Figure 7-39 shows the format of the SEND command structure for the Color OneScanner.

Figure 7-39 The SEND command structure for the Color OneScanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($2A) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) Reserved |  |  |  |  |
| 2 | ($01 or $03 or $80) Data type code |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | ($00) Data type qualifier (MSB) |  |  |  |  |  |  |  |
| 5 | ($02) Data type qualifier (LSB) |  |  |  |  |  |  |  |
| 6 | Transfer length (MSB) |  |  |  |  |  |  |  |
| 7 | Transfer length |  |  |  |  |  |  |  |
| 8 | Transfer length (LSB) |  |  |  |  |  |  |  |
| 9 | Control |  |  |  |  |  |  |  |

FIELD DESCRIPTIONS

Here is a list of the fields used in this command structure:

Operation code

The operation code for the SEND command is $2A.

Reserved 	These fields are reserved for future expansion. Set them to 0.

Data type code

This field indicates the type of parameter data to be transferred to the scanner. Set this field to one of the following values:

$01 Send 3-by-3 matrix for color correction

$03 Send gamma table

$80 Set calibration RAM

Data type qualifer

This field is always set to $02.

Transfer length

This field indicates the amount of parameter data to be sent.

If the transfer type is $80 (set calibration RAM), you must set the transfer length to 8100 bytes, to update the PSRAM calibration data for all three lines of sensors. Each single byte of data contains the new calibration data (CD) for each CD pixel sensor in ascending order. Set the transfer length to 18 for the 3-by-3 matrix, and to 768 for the gamma correction table.

### Halftone Parameter Page Description for the Apple Scanner and the OneScanner

The halftone parameter page, shown in Figure 7-40, defines the halftone matrix that the application program downloads to the scanner. The size of the parameter page varies according to the dimensions of the halftone matrix. The parameter page contains a single byte that indicates the matrix dimensions, followed by 1 byte of data for each matrix element. Thus, for a 4-by-4 matrix, the parameter page contains 17 bytes of data (the dimension byte followed by 16 bytes of matrix data), the parameter page for an 8-by-8 matrix contains 65 bytes, and so on.

Figure 7-40 The SEND command halftone parameter page

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | Matrix vertical size |  |  |  | Matrix horizontal size |  |  |  |
| 1 | Pïxel 0 |  |  |  |  |  |  |  |
| 2 | Pixel 1 |  |  |  |  |  |  |  |
| 3 | Pixel 2 |  |  |  |  |  |  |  |
| n + 1 | Pixel n |  |  |  |  |  |  |  |

Matrix vertical size, Matrix horizontal size

These fields indicate the dimensions of the matrix. The dimensions are encoded in a single byte, as follows: bits 4 through 7 contain the y dimension and bits 0 through 3 contain the x dimension. For a 4-by-4 matrix, set this field to $44. For an 8-by-8 matrix, set this field to $88. Figure 7-41 shows how the pixel numbers map onto a 4-by-4 matrix; pixels map similarly for an 8-by-8 matrix, except that there are eight rows, and each row contains eight elements.

Pixels 0 through n

Each of these pixel fields specifies a threshold level at which a particular pixel changes from black to white. Figure 7-41 shows the position of each pixel in a 4-by-4 matrix. The range of threshold values is from 0 to 255. In this range, 255 is the highest threshold setting, 0 is the lowest threshold setting, and 128 is the average threshold setting. A high threshold value results in most gray shades being changed to white. A low threshold value results in most gray shades being changed to black.

Figure 7-41 The halftone matrix pattern for a 4-by-4 matrix

| 0 | 1 | 2 | 3 |
| --- | --- | --- | --- |
| 4 | 5 | 6 | 7 |
| 8 | 9 | 10 | 11 |
| 12 | 13 | 14 | 15 |

### Halftone matrix patterns

The following tables contain Spiral and Bayer ordered dither matrix patterns of different sizes.

| Spiral 2-by-2 matrix 0; 1. 3; 2. | Bayer ordered dither 2-by-2 matrix 0; 3. 2; 1. |
| --- | --- |
| Spiral 3-by-3 matrix 0; 1; 2. 7; 8; 3. 6; 5; 4. | There is no Bayer ordered dither 3-by-3 matrix as Bayer matrices are square with a side length that is a power of 2. |
| Spiral 4-by-4 matrix 0; 1; 2; 3. 11; 12; 13; 4. 10; 15; 14; 5. 9; 8; 7; 6. | Bayer ordered dither 4-by-4 matrix 0; 12; 3; 15. 8; 4; 11; 7. 2; 14; 1; 13. 10; 6; 9; 5. |
| Spiral 8-by-8 matrix 0; 1; 2; 3; 4; 5; 6; 7. 27; 28; 29; 30; 31; 32; 33; 8. 26; 47; 48; 49; 50; 51; 34; 9. 25; 46; 59; 60; 61; 52; 35; 10. 24; 45; 58; 63; 62; 53; 36; 11. 23; 44; 57; 56; 55; 54; 37; 12. 22; 43; 42; 41; 40; 39; 38; 13. 21; 20; 19; 18; 17; 16; 15; 14. | Bayer ordered dither 8-by-8 matrix 0; 48; 12; 60; 3; 51; 15; 63. 32; 16; 44; 28; 35; 19; 47; 31. 8; 56; 4; 52; 11; 59; 7; 55. 40; 24; 36; 20; 43; 27; 39; 23. 2; 50; 14; 62; 1; 49; 13; 61. 34; 18; 46; 30; 33; 17; 45; 29. 10; 58; 6; 54; 9; 57; 5; 53. 42; 26; 38; 22; 41; 25; 37; 21. |

### Gamma Data Write Format for the Color OneScanner

Figure 7-42 shows the gamma data write format.

Figure 7-42 Gamma data write format

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | Red gamma value for grayscale value of 0 |  |  |  |  |  |  |  |
| 1 | Red gamma value for grayscale value of 1 |  |  |  |  |  |  |  |
| 2 | Red gamma value for grayscale value of 2 |  |  |  |  |  |  |  |
| 255 |  |  | Red gamma value for grayscale value of 255 |  |  |  |  |  |  |  |
| 256 |  |  | Green gamma value for grayscale value of 0 |  |  |  |  |  |  |  |
| 257 |  |  | Green gamma value for grayscale value of 1 |  |  |  |  |  |  |  |
| 258 |  |  | Green gamma value for grayscale value of 2 |  |  |  |  |  |  |  |
| 511 |  |  | Green gamma value for grayscale value of 255 |  |  |  |  |  |  |  |
| 512 |  |  | Blue gamma value for grayscale value of 0 |  |  |  |  |  |  |  |
| 513 |  |  | Blue gamma value for grayscale value of 1 |  |  |  |  |  |  |  |
| 514 |  |  | Blue gamma value for grayscale value of 2 |  |  |  |  |  |  |  |
| 767 |  |  | Blue gamma value for grayscale value of 255 |  |  |  |  |  |  |  |

### Color Correction Matrix for the Color OneScanner

Figure 7-43 shows the color correction matrix.

Figure 7-43 Color correction matrix

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | K0 byte 1 (Ka) |  |  |  |  |  |  |  |
| 1 | K0 byte 2 (Kb) |  |  |  |  |  |  |  |
| 2 | K1 byte 1 (Ka) |  |  |  |  |  |  |  |
| 3 |  | K1 byte 2 (Kb) |  |  |  |  |  |  |  |
| 4 |  | K2 byte 1 (Ka) |  |  |  |  |  |  |  |
| 5 |  | K2 byte 2 (Kb) |  |  |  |  |  |  |  |
| 6 |  | K3 byte 1 (Ka) |  |  |  |  |  |  |  |
| 7 |  | K3 byte 2 (Kb) |  |  |  |  |  |  |  |
| 8 |  | K4 byte 1 (Ka) |  |  |  |  |  |  |  |
| 9 |  | K4 byte 2 (Kb) |  |  |  |  |  |  |  |
| 10 |  | K5 byte 1 (Ka) |  |  |  |  |  |  |  |
| 11 |  | K5 byte 2 (Kb) |  |  |  |  |  |  |  |
| 12 |  | K6 byte 1 (Ka) |  |  |  |  |  |  |  |
| 13 |  | K6 byte 2 (Kb) |  |  |  |  |  |  |  |
| 14 |  | K7 byte 1 (Ka) |  |  |  |  |  |  |  |
| 15 |  | K7 byte 2 (Kb) |  |  |  |  |  |  |  |
| 16 |  | K8 byte 1 (Ka) |  |  |  |  |  |  |  |
| 17 |  | K8 byte 2 (Kb) |  |  |  |  |  |  |  |

The format for each byte is shown below. Ka corresponds to byte 1 of K0-8, and Kb corresponds to byte 2 of K0-8. The sign bit indicates plus or minus, MSB is the most significant bit, LSB the least significant bit.

|  | Ka |  |  | Kb |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Byte | Bit 15 | Bits 9 - 14 |  | Bit 8 | Bits 7 - 1 | Bit 0 |
| Ka and Kb | Sign | 0 |  | MSB | Data bits | LSB |

### PSRAM Data Write Format for the Color OneScanner

Figure 7-44 shows the PSRAM data write format.

Figure 7-44 PSRAM data write format

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | Calibration data for red CCD pixel sensor #1 |  |  |  |  |  |  |  |
| 1 | Calibration data for red CCD pixel sensor #2 |  |  |  |  |  |  |  |
| 2 | Calibration data for red CCD pixel sensor #3 |  |  |  |  |  |  |  |
| 2699 |  |  | Calibration data for red CCD pixel sensor #2700 |  |  |  |  |  |  |  |
| 2700 |  |  | Calibration data for green CCD pixel sensor #1 |  |  |  |  |  |  |  |
| 2701 |  |  | Calibration data for green CCD pixel sensor #2 |  |  |  |  |  |  |  |
| 2702 |  |  | Calibration data for green CCD pixel sensor #3 |  |  |  |  |  |  |  |
| 5399 |  |  | Calibration data for green CCD pixel sensor #2700 |  |  |  |  |  |  |  |
| 5400 |  |  | Calibration data for blue CCD pixel sensor #1 |  |  |  |  |  |  |  |
| 5401 |  |  | Calibration data for blue CCD pixel sensor #2 |  |  |  |  |  |  |  |
| 5402 |  |  | Calibration data for blue CCD pixel sensor #3 |  |  |  |  |  |  |  |
| 8099 |  |  | Calibration data for blue CCD pixel sensor #2700 |  |  |  |  |  |  |  |

## OBJECT POSITION ($31)

The OBJECT POSITION command allows the host computer to control the position of the scanner carriage assembly. The command provides options to park and unpark the carriage and to set the carriage to a specified position. Figure 7-45 shows the format of the OBJECT POSITION command structure.

Note

This command is valid only for the OneScanner and the Color OneScanner. The Apple Scanner does not support this command.

Figure 7-45 The OBJECT POSITION command structure

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($31) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) Reserved |  | Position function |  |  |
| 2 | Count (MSB) |  |  |  |  |  |  |  |
| 3 | Count |  |  |  |  |  |  |  |
| 4 | Count (LSB) |  |  |  |  |  |  |  |
| 5 - 8 | ($00) Reserved |  |  |  |  |  |  |  |
| 9 | Control |  |  |  |  |  |  |  |

FIELD DESCRIPTIONS

Here is a list of the fields used in this command structure:

Operation code

The operation code for the OBJECT POSITION command is $31.

Reserved	These fields are reserved for future expansion. Set them to 0.

Position type

This field specifies the action for the scanner to take. Valid values are as follows:

Unpark carriage $00

Instructs the scanner to move the carriage to the home position. If the carriage is already at the home position, the scanner returns a GOOD status. If the scanner cannot move the carriage to the home position, the scanner returns a CHECK CONDITION status and sets the sense data to VENDOR UNIQUE ($9).

Park carriage $01

Instructs the scanner to move the carriage to the shipment lock position. If the carriage is already at that position, the scanner returns a GOOD status.

Absolute position $02

Instructs the scanner to position the carriage at the scan line specified in the Count field. A 0 value causes the scanner to place the carriage at the beginning of the scan area. Other values must indicate a valid y-axis position in increments of 1/1200 of an inch. If the value of Count corresponds to an invalid scan line, the scanner returns a CHECK CONDITION status and sets the sense data to ILLEGAL REQUEST ($5).

Relative position $03

Instructs the scanner to move the carriage to a position relative to its current position. The value of the Count field indicates the direction and distance for the operation. Positive values move the carriage forward; negative values move the carriage backward. The distance is expressed in increments of 1/1200 of an inch. A 0 value does not move the carriage. If the value of Count yields an invalid scan line, the scanner returns a CHECK CONDITION status and sets the sense data to ILLEGAL REQUEST ($5).

Rotate object $04

This position function specifies that the object is rotated in an anti-clockwise direction expressed in thousandths of a degree. The count field specifies the number of units that the object is to be moved.

$05 - $07	The three remaining codes are reserved.

Count	This field specifies the distance for absolute and relative position requests. The scanner ignores this field for park and unpark requests.

## GET DATA BUFFER STATUS ($34)

The GET DATA BUFFER STATUS command allows the host computer to determine how much image data the scanner currently holds. The host computer can then decide whether to issue a READ command to retrieve the stored data. The scanner reports data availability for those windows that were defined with the DEFINE WINDOW PARAMETERS command and were specified in the current SCAN command. Figure 7-46 shows the format of the GET DATA STATUS command structure. The returned data blocks for each supported scanner are described later in this section.

Figure 7-46 The GET DATA STATUS command structure

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($34) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) Reserved |  |  |  | Wait |
| 2 - 5 | ($00) Reserved |  |  |  |  |  |  |  |
| 6 | Allocation length (MSB) |  |  |  |  |  |  |  |
| 7 | Allocation length |  |  |  |  |  |  |  |
| 8 | Allocation length (LSB) |  |  |  |  |  |  |  |
| 9 | Control |  |  |  |  |  |  |  |

FIELD DESCRIPTIONS

Here is a list of the fields and the bits used in this command structure:

Operation code
The operation code for the GET DATA BUFFER STATUS command is $34.

Reserved	These fields are reserved for future expansion. Set them to 0.

Wait	This bit indicates when the scanner returns data status to the host computer. If the value of the field is 1, the scanner waits for the quantity of data in the scanner’s internal memory to reach the limit set by the MODE SELECT command before the scanner responds with data. (This limit is set by the Buffer full ratio field of the disconnect-reconnect parameter page.) If the value is 0, the scanner responds immediately.

Allocation length
This field specifies the number of bytes that the host computer has allocated for returned data status. If the value of the Allocation length is 0, the scanner will not return data status (this is not an error condition). With any other value, the maximum number of bytes is transferred. The command terminates when the bytes specified by the Allocation length field have been transferred, or when all available status data bytes have been transferred to the host, whichever comes first.

### GET DATA BUFFER STATUS Return Structure Description for the Apple Scanner

Figure 7-47 shows the format of the GET DATA BUFFER STATUS return structure for the Apple Scanner.

Figure 7-47 The GET DATA BUFFER STATUS return structure for the Apple Scanner

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($00) Data buffer status length (MSB) |  |  |  |  |  |  |  |
| 1 | ($00) Data buffer status length |  |  |  |  |  |  |  |
| 2 | ($08) Data buffer status length (LSB) |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  | Block |

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | Window identifier |  |  |  |  |  |  |  |
| 1 | ($00) Reserved |  |  |  |  |  |  |  |
| 2 | Available data buffer (MSB) |  |  |  |  |  |  |  |
| 3 | Available data buffer |  |  |  |  |  |  |  |
| 4 | Available data buffer (LSB) |  |  |  |  |  |  |  |
| 5 | Filled data buffer (MSB) |  |  |  |  |  |  |  |
| 6 | Filled data buffer |  |  |  |  |  |  |  |
| 7 | Filled data buffer (LSB) |  |  |  |  |  |  |  |

FIELD DESCRIPTIONS

The parameter list header describes the length, in bytes, of the descriptor information that follows. Here are the parameter list fields used in this structure:

Data length	This field indicates the number of bytes of status data that follows, starting at byte 4. The value of this field does not include bytes 0 through 3. Each return structure is associated with a window descriptor. If the Apple bit is set in the DEFINE WINDOW PARAMETERS command, the scanner returns data status for only the first window descriptor. Data for secondary scan areas is returned together with the data for the primary scan area. The scanner indicates that a scan is complete and that all data has been transferred by setting the Data length field to 0 and sending no further status data.

Reserved	These fields are reserved for future expansion. Set them to 0.

Block	This field indicates the scanner buffer status. A value of 1 indicates that the scanner’s internal buffers are full and that the scanner must transfer all available scan data to the host computer before it can generate more scan data. The bit is also set to 1 when the scanner has reached the end of the scan, even though the buffer may not be full. A value of 0 indicates that the scanner is not currently blocked because of insufficient buffer space.

The data status descriptors contain information about defined scan windows. Each descriptor applies to a single window. Here are the data descriptors used in this structure:

Window identifier

This field identifies the window associated with this return structure. This value matches the window identifier in the window descriptor parameter of the DEFINE WINDOW PARAMETERS command.

Reserved	These fields are reserved for future expansion. Set them to 0.

Available data buffer

The available data buffer field indicates, in bytes, the amount of buffer available for transfers from the initiator. This field is valid only in scanners with the ability to accept data from an initiator for processing.

Filled data buffer

This field indicates the number of bytes of data that the scanner can send for the scan window identified by the Window identifier field. If the value is 0, it indicates that the scanner currently has no data available for the window (perhaps because it has not reached the window in its scanning process). This is not an error condition.

### GET DATA BUFFER STATUS Return Structure Description for the OneScanner and the Color OneScanner

Figure 7-48 shows the format of the GET DATA BUFFER STATUS return structure.

Figure 7-48 The GET DATA BUFFER STATUS return structure

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($00) Data buffer status length (MSB) |  |  |  |  |  |  |  |
| 1 | ($00) Data buffer status length |  |  |  |  |  |  |  |
| 2 | ($08) Data buffer status length (LSB) |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  | Block |

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | Window identifier |  |  |  |  |  |  |  |
| 1 | ($00) Reserved |  |  |  |  |  |  |  |
| 2 | Available data buffer (MSB) |  |  |  |  |  |  |  |
| 3 | Available data buffer |  |  |  |  |  |  |  |
| 4 | Available data buffer (LSB) |  |  |  |  |  |  |  |
| 5 | Filled data buffer (MSB) |  |  |  |  |  |  |  |
| 6 | Filled data buffer |  |  |  |  |  |  |  |
| 7 | Filled data buffer (LSB) |  |  |  |  |  |  |  |

FIELD DESCRIPTIONS

The parameter list header describes the length, in bytes, of the descriptor information that follows. Here are the parameter list fields used in this structure:

Data length	This field indicates the number of bytes of status data that follows, starting at byte 4. The value of this field does not include bytes 0 through 3. Each return structure is associated with a window descriptor. If the Apple bit is set in the DEFINE WINDOW PARAMETERS command, the scanner returns data status for only the first window descriptor. Data for secondary scan areas is returned together with the data for the primary scan area. The scanner indicates that a scan is complete and that all data has been transferred by setting the Data length field to 0 and sending no further status data.

Reserved	These fields are reserved for future expansion. Set them to 0.

Block	This field indicates the scanner buffer status. A value of 1 indicates that the scanner’s internal buffers are full and that the scanner must transfer all available scan data to the host computer before it can generate more scan data. The bit is also set to 1 when the scanner has reached the end of the scan, even though the buffer may not be full. A value of 0 indicates that the scanner is not currently blocked because of insufficient buffer space.

The data status descriptors contain information about defined scan windows. Each descriptor applies to a single window. Here are the data descriptors used in this structure:

Window identifier

This field identifies the window associated with this return structure. This value matches the window identifier in the window descriptor parameter of the DEFINE WINDOW PARAMETERS command.

Reserved	These fields are reserved for future expansion. Set them to 0.

Available data buffer

The available data buffer field indicates, in bytes, the amount of buffer available for transfers from the initiator. This field is valid only in scanners with the ability to accept data from an initiator for processing.

Filled data buffer

This field indicates the number of bytes of data that the scanner can send for the scan window identified by the Window identifier field. If the value is 0, it indicates that the scanner currently has no data available for the window (perhaps because it has not reached the window in its scanning process). This is not an error condition.

## UNKNOWN COMMAND $D3 ($D3)

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($D3) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) Reserved |  |  |  |  |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | Allocation length |  |  |  |  |  |  |  |
| 5 | Control |  |  |  |  |  |  |  |

## UNKNOWN COMMAND $D4 ($D4)

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($D4) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) Reserved |  |  |  |  |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | Allocation length |  |  |  |  |  |  |  |
| 5 | Control |  |  |  |  |  |  |  |

## UNKNOWN COMMAND $D5 ($D5)

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($D5) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) Reserved |  |  |  |  |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | Allocation length |  |  |  |  |  |  |  |
| 5 | Control |  |  |  |  |  |  |  |

## UNKNOWN COMMAND $D6 ($D6)

| Bit | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Byte |  |  |  |  |  |  |  |  |
| 0 | ($D6) Operation code |  |  |  |  |  |  |  |
| 1 | Logical unit number |  |  | ($00) Reserved |  |  |  |  |
| 2 | ($00) Reserved |  |  |  |  |  |  |  |
| 3 | ($00) Reserved |  |  |  |  |  |  |  |
| 4 | Allocation length |  |  |  |  |  |  |  |
| 5 | Control |  |  |  |  |  |  |  |

# The programming part

Open	=	$2010	; GS/OS reference

Read	=	$2012

Close	=	$2014

DInfo	=	$202C

DStatus = 	$202D

DControl = 	$202E

devSCANNER =	$001A	; GS/OS (Device) reference

attrFixed =	$4000	; Memory Manager

## Find a scanner device

The routine goes through the list of connected devices and exits with the carry clear when it finds a scanner device. The string for the first device is 'APPLESCSI.SCANNER01.00'

lda	#1		; start with device 1

sta	proDINFO+2

]lp	jsl	GSOS		; do a DInfo

dw	DInfo

adrl	proDINFO

bcc	found		; we have a device

cmp	#$0011		; do we have more devices?

bne	loop

rts			; no, return with carry set

loop	inc	proDINFO+2	; check next device

bra	]lp

found	lda	proDINFO+20	; is it a scanner?

cmp	#devSCANNER

bne	loop		; no

rts			; yes, return with carry clear

*---

proDINFO

dw	8		; 00 pcount

ds	2		; 02 device num

adrl	devINFO	; 04 device name

ds	2		; 08 characteristics

ds	4		; 0A total blocks

ds	2		; 0E slot number

ds	2		; 10 unit number

ds	2		; 12 version

ds	2		; 14 device id

devINFO

dw	$36		; buffer size (GS/OS output buffer)

devNAME

ds	$34		; device name (GS/OS string)

## Identify the scanner brand and model

Now that we have our device ID, we can use it to call the DControl Inquiry command that will return the information we want.

The scanner brand (Vendor identification) is a left-justified string at inqBUFFER[8..15]. The scanner model (Product identification) is a left-justified string at inqBUFFER[16..31]. See "How to identify" for more information.

lda	proDINFO+2	; get the device number

sta	proINQUIRY+2

jsl	GSOS		; get the Inquiry table data

dw	DStatus

adrl	proINQUIRY

rts

*---

proINQUIRY

dw	5		; 00 pcount

ds	2		; 02 device num

dw	$8012		; 04 status code

adrl	inqLIST	; 06 status list

adrl	240		; 0A request count

ds	4		; 0E transfer count

inqLIST

ds	2		; always 0000

hex	12		; 00 command

ds	3		; 01 3 zeroes

dfb	240		; 04 length

ds	7		; 05 7 zeroes

adrl	inqBUFFER	; 12 result buffer

inqBUFFER

ds	240		; result buffer, see Table 45 description for more information

## Preparing the scanner

The first action is to open the device to let the Character FST communicate with the scanner. We also set the scanner in WAIT mode so that the READ call will return to the caller once all data is read.

jsl	GSOS		; open the device for the CHAR.FST

dw	Open

adrl	proOPEN

lda	proDINFO+2	; get the device number

sta	proINQUIRY+2

jsl	GSOS		; set the device in WAIT mode

dw	DControl

adrl	proWAIT

rts

*---

proOPEN

dw	2		; 00 pcount

ds	2		; 02 ref num

adrl	devNAME	; 04 pathname

proWAIT

dw	5		; 00 pcount

ds	2		; 02 device num

dw	$0004		; 04 control code

adrl	waitLIST	; 06 control list

adrl	2		; 0A request count

ds	4		; 0E transfer count

waitLIST

dw	$0000		; $0000: wait mode / $8000: no-wait mode

## Scanning a page

Scanning a page means preparing the window that defines the scanning area. Then, we must reserve memory for the resulting picture. Finally, we scan the page.

* 1. Prepare the scanning window

* 2. We set the scanning area

lda	proDINFO+2	; get the device number

sta	proWINDOW+2

jsl	GSOS		; define the scanning window

dw	DControl

adrl	proWINDOW

* 3. We reserve memory

pha			; space for result

pha

PushLong bufferSIZE	; the number of bytes to reserve

PushWord myID		; my user ID

PushWord #attrFixed	; $4000

PushLong #0		; location pointer

_NewHandle

phd			; deference the handle

tsc			; to get the pointer

tcd

lda	[3]

sta	proREAD+4

ldy	#2

lda	[3],y

sta	proREAD+6

pld

PullLong myHANDLE

* 4. We scan the page

lda	proOPEN+2	; get the ref number

sta	proREAD+2

sta	proCLOSE+2

jsl	GSOS		; read the data from the scanner

dw	Read

adrl	proREAD

jsl	GSOS		; close the file

dw	Close

adrl	proCLOSE

rts			; we're done!

*---

myID

ds	2		; my (your) Memory Manager user ID

bufferSIZE

ds	4		; size of data to scan

myHANDLE

ds	4		; handle to the scanned area

proWINDOW

proREAD

dw	4		; 00 pcount

ds	2		; 02 ref num

ds	4		; 04 data buffer

ds	4		; 08 request count

ds	4		; 0A transfer count

proCLOSE

dw	1		; 00 pcount

ds	2		; 02 ref num

# References

## Halftoning Theory for ASCII Art

http://caca.zoy.org/study/

## Programmer's Guide to Apple Scanners Second Edition

by Apple Computer, Inc. 1995

Developer Technical Publications

## SCSI-2 Protocol Reference

https://www.staff.uni-mainz.de/tacke/scsi/SCSI2.html

See Chapter 15 for scanner devices

## The Complete Scanner Toolkit Macintosh Edition

by David D. Busch

the Business One Irwin Desktop Publishing Library

https://vintageapple.org/macbooks/pdf/The_Complete_Scanner_Toolkit_Macintosh_Edition_1992.pdf

# Glossary

| Byte | Indicates an 8-bit construct. |
| --- | --- |
| Command descriptor block (CDB) | The structure used to communicate commands from an initiator to a target. |
| Host adapter | A device which connects between a host system and the SCSI bus. The device usually performs the lower layers of the SCSI protocol and normally operates in the initiator role. This function may be integrated into the host system. |
| Initiator | An SCSI device that requests an I/O process to be performed by another SCSI device (a target). |
| I_T_L nexus | A nexus which exists between an initiator, a target, and a logical unit. This relationship replaces the prior I_T nexus. |
| Logical unit | A physical or virtual peripheral device addressable through a target. |
| Logical unit number | An encoded three-bit identifier for the logical unit. |
| Nexus | A relationship that begins with the establishment of an initial connection and ends with the completion of the I/O process. The relationship may be restricted to specify a single logical unit or target routine by the successful transfer of an IDENTIFY message. The relationship may be further restricted by the successful transfer of a queue tag message. |
| Optional | The referenced item is not required to claim compliance with this standard. Implementation of an optional item must be as defined in this standard. |
| Reserved | Identifies bits, fields, and code values that are set aside for future standardization. |
| Scanner | Any graphic input device that converts printed matter into bit (binary digital) data. |
| Status | One byte of information sent from a target to an initiator upon completion of each command. |
| Target | An SCSI device that performs an operation requested by an initiator. |
| Vendor-specific | Something (e.g. a bit, field, code value, etc.) that is not defined by this standard and may be used differently in various implementations. |